# Chinese EV Charging Standards Abroad: GB/T vs CCS2 vs CHAdeMO vs NACS Compatibility

# SEO Metadata

SEO Title: GB/T vs CCS2 vs CHAdeMO vs NACS: Chinese EV Export Compatibility

Meta Description: Will a Chinese-market EV charge in Europe, Japan or North America? GB/T 20234.3/27930 explained, the connector map by region, export-version inlets, adapters and the ChaoJi direction.

Suggested URL: /guides/chinese-ev-charging-standard-compatibility/

H1: Will a Chinese EV Charge Overseas? GB/T, CCS2, CHAdeMO and NACS Compatibility Explained

Primary Keyword: GB/T CCS2 CHAdeMO charging standard export compatibility

Secondary Search Terms: Chinese EV export charging adapter, GB/T 20234.3 DC fast charge, GB/T 27930 protocol, CCS2 export version EV, ChaoJi standard

Internal Link Suggestions: /vehicles/byd-yuan-plus/ ; /guides/right-hand-drive-chinese-cars/ ; /guides/used-chinese-ev-inspection/

Image Suggestions: world connector-standard map; GB/T vs CCS2 inlet comparison; factory export-version inlet; adapter compliance warning

ALT Suggestions: "World map of DC fast-charging connector standards"; "GB/T and CCS2 charging inlets side by side"; "Chinese EV export-version CCS2 inlet"

# Why the Connector Decides Whether the Car Is Usable

A Chinese-domestic EV that passes customs can still be effectively unusable if its charging inlet does not match the public charging network. Charging compatibility is a physical connector plus communication-protocol problem, not a brand preference, and it must be resolved before the vehicle is specified for export — retrofitting an inlet after arrival is costly and sometimes non-compliant.

# The Standards Map (DC Fast Charging)

| Standard | Primary deployment | Notes |
| --- | --- | --- |
| GB/T 20234.3 (DC) + GB/T 27930 (CAN communication) | China (domestic Chinese vehicles) | GB/T 20234.3-2023 raises the upper limit to 1500 V / 800 A (cross-checked industry sources) |
| CCS2 (Combo 2) | Europe and many export markets | Combined AC/DC inlet; dominant in the EU |
| CCS1 (Combo 1) | North America | Regional variant of CCS |
| CHAdeMO | Japan and selected markets | Originated in Japan |
| NACS | North America | Deploying across North America (2026 rollout timing not officially verified here) |

These DC connectors are physically mutually incompatible: you cannot plug a GB/T gun into a CCS2 socket. The AC (slow/overnight) inlet likewise differs by market and must be checked separately — a car may fast-charge on one standard while its AC inlet still needs attention.

# What "Chinese-Market GB/T" Means in Practice

Domestic Chinese EVs generally ship with GB/T DC charging and the GB/T 27930 CAN-based communication handshake. When the destination network predominantly runs CCS2 (Europe), CHAdeMO (Japan) or NACS/CCS1 (North America), this creates the compatibility bottleneck an importer must design around. Crucially, GB/T and CCS2 have no direct physical compatibility — an adapter is required to bridge them (per an EVSE industry comparison), and an adapter must also make the communication handshake work, not merely fit mechanically.

# Three Ways to Solve It — in Order of Preference

Order the factory export version with the destination inlet. Many Chinese manufacturers build export-market variants fitted with the target connector (e.g., CCS2) rather than the domestic GB/T inlet. This is the cleanest route because inlet, onboard software and certification are aligned. Confirm the exact connector per VIN/model on the brand's official export configuration — domestic and export versions of the "same" model differ.

Use a certified adapter (GB/T ↔ destination). Where a factory export version is unavailable, an adapter bridges physical/communication differences. Treat this as a compliance question, not just hardware: some markets restrict adapter use, and adapter certification/legal status was not captured from an official source — verify locally and carry the adapter's certification documents. Adapter charging speed and handshake reliability should be tested before fleet rollout.

Solve it on the infrastructure side. Depot/fleet operators can install GB/T-capable chargers at their own premises, removing dependence on the public network — viable for closed fleets, not for retail customers who rely on public stations.

# ChaoJi: the Future Direction, Not Today's Default

The China–Japan ChaoJi ultra-fast-charging project is designed so that a common physical interface is compatible across GB/T, CHAdeMO and CCS systems, and is regarded as a future DC standardisation direction (per an official CHAdeMO Association document). It is forward-looking context: do not assume a current production GB/T vehicle already benefits from ChaoJi — verify model support officially before using it as a selling point.

# What AutoBridge Adds Beyond a Connector Chart

A standards chart tells you GB/T differs from CCS2; it does not tell you whether this VIN will charge. AutoBridge records the physical inlet, handshake protocol and onboard-charger rating as one VIN-bound record, distinguishes a hardware adapter (mechanical only) from a protocol gateway (GB/T 27930 handshake) in the purchase file, and verifies the adapter's legality and warranty effect in the destination before deposit rather than after arrival.

# Per-Vehicle Verification Matrix

For every model/trim you export, record in a sheet:

Domestic inlet: GB/T DC + AC inlet type

Available factory export inlet(s): CCS2 / CHAdeMO / NACS / CCS1 by VIN

Communication protocol and whether the export firmware supports the destination handshake

If adapter-based: adapter model, certification, max current/voltage and local legality

Public-network reality in the destination (dominant DC standard; AC socket standard)

Warranty implications of any inlet/adapter modification

# Before Payment

Confirm the destination's dominant DC and AC connector standards.

Obtain the brand's official export-version connector confirmation for the exact VIN — do not infer from the domestic spec.

If relying on an adapter, confirm local legality/certification and test a real fast-charge session.

For fleets, decide whether depot chargers remove public-network dependence.

Treat ChaoJi claims as future-facing unless officially confirmed for the unit.

# Frequently Asked Questions

Can a Chinese GB/T EV charge directly on European CCS2? No — GB/T and CCS2 are physically incompatible; you need a factory CCS2 export version or a suitable, locally compliant adapter. What is GB/T 27930? It is the CAN-based communication protocol used alongside the GB/T 20234.3 DC connector in China; the handshake matters as much as the plug shape. Is an adapter a permanent solution? It can bridge the gap, but adapter legality varies by market and speed/reliability should be tested; the factory export inlet is preferred. Do Chinese brands sell CCS2 versions? Many build export variants with the destination connector — confirm per model/VIN on official export configuration rather than assuming. Does ChaoJi make all connectors compatible now? ChaoJi is a designed-for-compatibility future direction; current production cars still need per-model confirmation.

# Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| ChaoJi standard presentation (official) | CHAdeMO Association (standards body) | CN/JP/Global | https://www.chademo.com/wp2016/wp-content/uploads/ChaoJi202006/ChaoJi_Presenataion_EN.pdf | 2026-09-02 | VERIFIED | ChaoJi designed compatible with GB/T/CHAdeMO/CCS |
| Charging-standard certification paths | Huayu Testing (certification body) | Global | http://www.huayutest.com/zixun/87747.html | 2026-09-02 | CROSS_CHECKED | CHAdeMO/CCS regional deployment, certification differences |
| Charging connector standards | cehome (industry media) | CN | https://m.cehome.com/news/20260809/389612.shtml | 2026-09-02 | CROSS_CHECKED | GB/T 20234.3-2023 1500V/800A, GB/T 27930, ChaoJi |
| GB/T, CCS2, Type 2, NACS, CHAdeMO compared | evse-chargers.com (industry) | Global | https://www.evse-chargers.com/news/understanding-ev-charging-standards-gb-t-ccs2-type-2-nacs-and-chademo-compared-283319.html | 2026-09-02 | CROSS_CHECKED | GB/T↔CCS2 requires adapter; compatibility matrix |
| Guide to global EV charging standards | MARUIKEL (industry) | Global | https://www.maruikel.com/es/blog/guide-to-global-ev-charging-standards-type-1-type-2-ccs-chademo-gbt.html | 2026-09-02 | CROSS_CHECKED | Domestic GB/T vs export-version destination connector |
| GB/T-to-CHAdeMO adapter B2B guide | Electric Auto China (industry) | Global | https://www.electricautochina.com/comprehensive-b2b-guide-to-gbt-to-chademo-adapters-for-exported-chines/ | 2026-09-02 | CROSS_CHECKED | Export compatibility bottleneck |

Confidence note (AutoBridge standard): standard-level facts are VERIFIED/CROSS_CHECKED (CHAdeMO Association is a standards body). Per-model export connectors, adapter legality by country and NACS rollout timing were not captured and must be confirmed per VIN and per destination authority.

# Editorial Review

Author / reviewer: [AutoBridge Export Sourcing Team](/authors/autobridge-export-sourcing-team/) · method per our [Editorial Policy](/about/editorial-policy/)

Last reviewed: 2026-09-02 · Second-round revision: 2026-09-03 (confidence relabelled; increment block added)

Reference market: Global (China export side; EU/JP/NA deployment)

Verification method: Standards-body document plus cross-checked industry sources; model-specific connectors left to official per-VIN confirmation

Quality gates: EDITORIAL_QA_PASS=PASS · FACT_SOURCE_PASS=PASS · SEO_PASS=PASS · CODEX_REVIEW_PASS=PENDING (independent Codex review) · PUBLISH_APPROVED=FALSE until Codex sign-off