# Chery Fulwin T9 (风云T9): a C-DM Plug-in Hybrid Mid-Size SUV — China Specs for Export

# SEO Metadata

SEO Title: Chery Fulwin T9 C-DM PHEV Specs & Export Guide (China Reference)

Meta Description: Chinese-market Chery Fulwin T9 C-DM plug-in hybrid SUV — 1.5T dedicated engine, 150 kW front motor, 32.66 kWh battery, 120–220 km CLTC, 5/7 seats, and exporter checks.

Suggested URL: /vehicles/chery-fulwin-t9/

H1: Chery Fulwin T9 C-DM: Chinese-Market PHEV SUV Specs and Export Verification

Primary Keyword: chery fulwin t9 specs export

Secondary Search Terms: Fulwin T9 C-DM, 风云T9 plug-in hybrid, Chery T9 32.66kWh, Fulwin T9 7-seat, Chinese PHEV SUV export

Internal Link Suggestions: /guides/ev-shipping-un3556-imdg-compliance/ ; /vehicles/byd-qin-l-dm-i/ ; /guides/export-vehicle-document-package/

Image Suggestions: Fulwin T9 three-quarter; charging port; 5 vs 7 seat layout; 1.5T C-DM badge; cargo area

ALT Suggestions: "Chery Fulwin T9 C-DM plug-in hybrid SUV"; "Fulwin T9 32.66kWh battery CLTC range"

# Buyer Context: a PHEV Family SUV With a Long-Battery Version

The Fulwin T9 is Chery's C-DM mid-size plug-in hybrid SUV, offered in five- and seven-seat forms. Two order errors dominate this model: mixing the standard T9 with the stretched T9L, and quoting electric range without naming the battery. This page documents only the standard Fulwin T9 (REFERENCE MARKET: CHINA); the longer T9L (roughly 4870 mm / 2920 mm wheelbase) is a different body and is deliberately excluded. An OEM configuration page was captured for this model, which strengthens the spec base.

# PHEV Powertrain: Dedicated Engine + Motor + Single-Speed DHT

| Item | Fulwin T9 C-DM (China reference) |
| --- | --- |
| Engine | 1.5 T dedicated hybrid petrol (115 kW / 220 N·m engine-only; China-6b) |
| Front motor | 150 kW / 310 N·m |
| Combined system output | about 265 kW / 530 N·m |
| Hybrid transmission | Single-speed DHT |
| Long-range battery | 32.66 kWh |
| Electric range | 120–220 km CLTC (version-dependent; a WLTC figure of about 160 km is also cited for one version) |
| Combined range | about 1400 km WLTC, manufacturer-claimed — label as a factory claim, not measured |
| Charging | DC about 20 min for 30–80%; AC slow charge about 3 h (single-source; confirm per OEM) |

For a PHEV the exporter fixes engine, motor, battery and charging together. The 1.5T here is a hybrid-dedicated engine; its 115 kW engine-only figure is not the system output — keep the two apart on any spec line.

# Body, Seating and Cargo

Length / width / height: 4795 / 1930 / 1738 mm (some sources list 1741 mm height by trim)

Wheelbase: 2770 mm

Seats: 5 or 7 — state which on the order

Cargo: 650 L, about 1775 L with seats folded (single-source)

# Standard T9 vs Stretched T9L — Do Not Merge

This page: standard T9, 4795 mm / 2770 mm wheelbase.

Excluded: T9L, about 4870 mm with a 2920 mm wheelbase. It is a longer wheelbase vehicle; mixing its dimensions or seating claims into a standard-T9 order is a configuration error.

Range steps and seat layouts vary by trim; the 120–220 km spread is not a single number for one car.

# Overseas Buyer Verification Before Payment

Body version — standard T9, never silently substituted with T9L.

Battery capacity and range cycle — 32.66 kWh long-range vs smaller versions; label CLTC vs WLTC and keep manufacturer combined-range claims attributed.

Seat count (5 vs 7) matched to the VIN.

Charging connector and DC/AC hardware vs the destination standard; PHEV battery shipping documents (UN38.3 summary) per the IMDG 42-24 guide.

Emission/hybrid registration class and full document set per the export document-package checklist.

# Request a Current Export Quotation

No export price is published. The Chinese 2024 guide range (roughly RMB 129,900–169,900) is a domestic, time-sensitive reference, not FOB/CIF. Enquire with Destination Country · Quantity · Battery/Seat Trim · LHD/RHD · Destination Port · Incoterm.

# Frequently Asked Questions

Is the Fulwin T9 the same as the T9L? No; the T9L is a longer-wheelbase vehicle (about 4870 mm / 2920 mm) and is excluded from this standard-T9 sheet. What battery does the long-range T9 use? A 32.66 kWh pack for the documented long-range versions, with 120–220 km CLTC electric range depending on trim. Is the 1400 km range measured? It is a manufacturer-claimed WLTC combined figure; present it as a factory claim, not a test result. Five or seven seats? Both are offered; specify on the order and match to the VIN.

# Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| Fulwin T9 official configuration table | Chery Fulwin (OEM) | CN | https://fulwin.chery.cn/vehicles/jac-t9-hunter/config/ | 2026-09-04 | VERIFIED (OEM) | Battery, dimensions, seats, charging, versions |
| Fulwin T9 hands-on (size/model year) | Dongchedi | CN | https://m.dongchedi.com/article/7506038314424730131 | 2026-09-04 | CROSS_CHECKED | Dimensions, model-year boundary |
| Fulwin T9 120 long-range flagship config | Autohome config | CN | https://m.autohome.com.cn/config/spec/70272.html | 2026-09-04 | CROSS_CHECKED | 32.66 kWh, electric range, motor |
| Fulwin T9 battery & EV-range analysis | Autohome·Chejiahao | CN | https://chejiahao.m.autohome.com.cn/pingan/chejiahao/detailinfo/26113006 | 2026-09-04 | SINGLE_SOURCE | Battery, CLTC/WLTC range, charge times |
| Fulwin T9 launch & dimensions | Sohu Auto database | CN | http://db.m.auto.sohu.com/model_7322/a/1019838375_120590677 | 2026-09-04 | CROSS_CHECKED | Mid-size PHEV SUV size, domestic price |

Confidence note: OEM configuration page captured (VERIFIED anchor); charge times and cargo volume are single-source; combined range is a manufacturer claim. Standard T9 only — T9L excluded.

# Editorial Review

Author: AutoBridge Export Editorial Team · [Sourcing Team](/authors/) · method per our [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-04 · Fact sheet version: v1.1-2026-09-04

Reference market: China, standard Fulwin T9 C-DM (Chinese-market reference; CLTC/WLTC labelled; not a global spec)

Verification method: OEM config anchored; standard vs long-wheelbase isolated; factory claims attributed

Transparency: AI_ASSISTED=true · HUMAN_REVIEWED=false · evidence.first_hand=false · PUBLISH_APPROVED=false

Quality gates: EDITORIAL_QA_PASS=PASS · FACT_SOURCE_PASS=PASS · SEO_PASS=PASS · CODEX_REVIEW_PASS=PENDING · PUBLISH_APPROVED=FALSE until independent sign-off

Tags: #CheryFulwinT9 #CDM #PHEVSUV #ChinaReferenceSpec #ExportBuyerGuide