# Farizon Xingxiang V6E Electric Cargo Van: Battery-Year Lock, 6 m³ Load Bay and a Documented Türkiye Export

## SEO Metadata

SEO Title: Farizon V6E Electric Cargo Van Specs & Export Buyer Notes

Meta Description: Chinese-market Farizon (Geely) Xingxiang V6E electric cargo van — 60 kW/220 N·m motor, 6 m³ cargo volume, 35.2 kWh/260 km base vs newer 51 kWh build, 90 km/h limit, and a recorded EU-spec Türkiye shipment.

Suggested URL: /vehicles/farizon-xingxiang-v6e-specs/

H1: Farizon Xingxiang V6E (Chinese-Market): Electric Cargo Van Specs and Export Verification

Primary Keyword: farizon v6e electric cargo van specs

Secondary Search Terms: Xingxiang V6E range, Farizon electric delivery van, 6m3 electric van, V6E 35.2 kWh, Farizon Türkiye export, electric city logistics van

Internal Link Suggestions: /vehicles/saic-maxus-v90-specs/ ; /vehicles/byd-dolphin/ ; /guides/ev-shipping-un3171-imdg-compliance/

Image Suggestions: cargo bay with dimensions; side loading door; motor/battery layout; leaf-spring rear axle; EU-spec export batch

ALT Suggestions: "Farizon V6E six cubic metre electric cargo van"; "V6E battery grades and range"

## Buyer Context: Lock the Battery Year Before Anything Else

The Xingxiang V6E is Farizon's (Geely's commercial-vehicle brand) battery-electric urban delivery van. The single biggest spec risk is model year: the base build uses a 35.2 kWh pack while a newer build uses about 51 kWh, and range changes accordingly. A purchase order that says only "V6E" can be filled with either. This page covers the Chinese-market V6E (REFERENCE MARKET: CHINA) and keeps the base and newer builds separate.

## Driveline and Cargo (electric-van focus)

| Item | V6E China reference |
| --- | --- |
| Motor | Wolong-ZF **TZ185XS022** permanent-magnet, **60 kW / 220 N·m** |
| Top speed | **90 km/h (electronically limited)** |
| Cargo volume | **6 m³** (cargo bay ~1.6 m long reference) |
| Rear suspension | Rigid axle with **leaf springs** (load-oriented) |
| Base battery/range | **35.2 kWh / 260 km working-condition range** |
| Newer build | A **larger-battery Xuanwu build is listed in Chinese databases**, but its exact capacity/range is not asserted here — confirm on the MIIT catalog and the VIN build sheet (TIME_SENSITIVE) |

For a logistics van, buyers should weigh cargo volume, payload (confirm on the homologated code), the load-oriented leaf-spring rear axle and the 90 km/h limit against route speed needs — this is a city/last-mile vehicle, not a long-haul truck.

## Battery-Year Discipline

Base (this page's anchor): 35.2 kWh, ~260 km working-condition range (cross-checked).

Newer build: a larger Xuanwu battery build exists in Chinese listings; do not quote a fixed number — confirm capacity and range on the MIIT catalog and by VIN (time-sensitive).

Quote pack kWh and range and range cycle/condition together; never attach the newer 51 kWh number to a base VIN. Confirm AC/DC charging provision and charge time per build.

## Documented Export Record (fact, not an alias)

China Transport News (an industry news outlet) reports that in February 2024 an initial batch of EU-specification V6E vans was exported to Türkiye, described as Farizon's first entry into that market, in 6 m³ form. This is an export-delivery record: it confirms a European-standard build was shipped, but it does not make the Türkiye EU-spec van identical to the China 35.2/51 kWh builds — treat export configuration separately and request its spec sheet.

## What Importers Should Verify Before Payment

Confirm the electric cargo van's drive-axle arrangement on the certificate; the captured Chinese reference data does not fix it, so do not default to front drive for a load-carrying chassis.

Battery build (35.2 vs ~51 kWh) and range cycle matched to VIN/model year.

Payload, GVW, cargo volume and rear-axle on the homologated code; leaf-spring ride vs cargo needs.

Charge interface (GB/T on China builds) vs destination standard; AC/DC provision.

EU-spec vs China-spec differences for any Türkiye/Europe-bound order (lighting, conformity, software).

90 km/h speed limit suitability; UN38.3/MSDS and under IMDG Amendment 42-24 (mandatory from 2026-01-01) a lithium-ion-battery-powered vehicle is classified UN3556, not the legacy generic UN3171; apply documentation/SOC details per the current EV shipping Fact Sheet and the carrier (see the EV ocean-compliance guide) EV shipping documents per the current Fact Sheet; cluster/telematics language.

## Request a Current Export Quotation

No static export price and no China MSRP were captured; none is invented. Enquire with Destination Country · Battery Build · Quantity · LHD/RHD · Destination Port · Incoterm.

## Frequently Asked Questions

What is the V6E's cargo capacity? A 6 m³ cargo bay (about 1.6 m long reference), with a load-oriented leaf-spring rear axle.

How far does it go on a charge? The reference base pack is 35.2 kWh at about 260 km (working condition); a larger-battery build is listed but its exact figure must be confirmed on the catalog and VIN rather than quoted.

What is the top speed? 90 km/h, electronically limited — a city/last-mile characteristic.

Has the V6E been exported? Yes — an EU-spec batch was recorded shipped to Türkiye in Feb 2024; that export build is not automatically identical to the China-spec van.

Which motor does it use? A Wolong-ZF TZ185XS022 permanent-magnet motor, 60 kW/220 N·m.

## Image Record

IMAGE_ASSET_PATH: none secured in repository

ORIGINAL_IMAGE_URL: not captured

SOURCE_PAGE: not captured

RIGHTS_HOLDER: unconfirmed

LICENSE_OR_USAGE_BASIS: none secured — no third-party image may be published until rights are cleared

CHECKED_DATE: 2026-09-05

MODEL_TOPIC_MATCH: must match the exact model/version (or the guide topic) and reference market above

IMAGE_RIGHTS_STATUS: FAIL (no licensed asset captured; a placeholder or “retain old image” note is not accepted)

ALT by language:

EN: AutoBridge export-buyer reference — Farizon Xingxiang V6E, battery-electric vehicle / cargo van

FR: Référence AutoBridge pour acheteurs export — Farizon Xingxiang V6E, véhicule 100 % électrique / fourgon

DE: AutoBridge-Referenz für Exportkäufer — Farizon Xingxiang V6E, batterieelektrisches Fahrzeug / Kastenwagen

ES: Referencia AutoBridge para compradores de exportación — Farizon Xingxiang V6E, vehículo 100 % eléctrico / furgoneta de carga

PT: Referência AutoBridge para compradores de exportação — Farizon Xingxiang V6E, veículo 100 % elétrico / furgão de carga

JA: AutoBridge 輸出バイヤー向けリファレンス｜Farizon Xingxiang V6E, 純電気自動車 / 貨物バン

KO: AutoBridge 수출 바이어 참고 자료｜Farizon Xingxiang V6E, 순수 전기차 / 카고 밴

VI: Tài liệu tham khảo AutoBridge cho người mua xuất khẩu — Farizon Xingxiang V6E, xe thuần điện / xe van chở hàng

TH: เอกสารอ้างอิง AutoBridge สำหรับผู้ซื้อเพื่อการส่งออก — Farizon Xingxiang V6E, รถยนต์ไฟฟ้า 100% / รถตู้บรรทุก

ID: Referensi AutoBridge untuk pembeli ekspor — Farizon Xingxiang V6E, kendaraan listrik murni / van kargo

AR: مرجع AutoBridge لمشتري التصدير — Farizon Xingxiang V6E, مركبة كهربائية بالبطارية / شاحنة فان للبضائع

ZH: AutoBridge 出口采购参考｜Farizon Xingxiang V6E, 纯电动车 / 厢式货车

## Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| V6E reveal (motor/0–50/speed limit) | Truck Home / Toutiao | CN | http://m.toutiao.com/group/7064810099012895240/ | 2026-09-03 | CROSS_CHECKED | 60 kW/220 N·m, 90 km/h limit, leaf-spring axle |
| V6E EU-spec export to Türkiye (6 m³) | China Transport News (industry news media) | CN media (export record: TR) | https://www.zgjtb.com/m/2024-02/01/content_398656.html | 2026-09-03 | CROSS_CHECKED | Feb-2024 EU-spec Türkiye batch, 6 m³ (news record, not OEM/government; not VERIFIED) |
| Light-commercial report (35.2 kWh/260 km/6 m³) | Sohu Auto | CN | https://m.sohu.com/a/931378694_121823499/ | 2026-09-03 | CROSS_CHECKED | Base battery/range, cargo volume |
| Road motor-vehicle manufacturers & products catalog — public query system | MIIT, China Ministry of Industry and Information Technology | CN | https://service.miit-eidc.org.cn/miitxxgk/gonggao_xxgk/index_ggcp.html | 2026-09-05 | VERIFIED | Verify the manufacturer, approved model and homologation (公告) catalog entry before ordering or export |
| GB 16735-2019 Road vehicles — Vehicle identification number (VIN) requirements | SAMR national standards full-text platform | CN | https://openstd.samr.gov.cn/bzgk/std/newGbInfo?hcno=E2EBF667F8C032B1EDFD6DF9C1114E02 | 2026-09-05 | VERIFIED | VIN structure and nameplate data fields used in the buyer's VIN/nameplate verification |
| GB 7258 motor-vehicle safety technical conditions (GB 7258-2017 current; GB 7258-2026 applies from 2027-07-01) | SAMR, State Administration for Market Regulation | CN | https://www.samr.gov.cn/xw/sj/art/2026/art_9b2b3fa425e3461e8dc3bd270b410ceb.html | 2026-09-05 | VERIFIED | Baseline in-China vehicle safety technical conditions and the current/revision timeline |
| GB 38031 traction battery safety requirements (GB 38031-2025; new type applications mandatory from 2026-07-01) | SAMR national standards full-text platform | CN | https://std.samr.gov.cn/gb/search/gbDetailed?id=31D72E7FB9BF92EDE06397BE0A0AC2E0 | 2026-09-05 | VERIFIED | Mandatory traction-battery safety requirement basis for the electrified powertrain |
| GB 18384 electric vehicle safety requirements (GB 18384-2025) | SAMR national standards full-text platform | CN | https://std.samr.gov.cn/gb/search/gbDetailed?id=473DC30DBF4CC20FE06397BE0A0AD08C | 2026-09-05 | VERIFIED | Electric-vehicle electrical/functional safety requirement basis |

*Confidence note: the Türkiye export record is CROSS_CHECKED industry-news reporting (news/industry media is not VERIFIED — only OEM/government/regulator/standards/formal technical documents reach VERIFIED); treated strictly as a delivery record, not a model-alias proof. Base motor/range are cross-checked; the newer 51 kWh build is time-sensitive. China GB/T and EU-spec builds are kept distinct per MODEL_ALIAS / SOURCE_SCOPE rules.*

## Editorial Review

Author / reviewer: [AutoBridge Export Editorial Team](/authors/) · method per our [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-05

Reference market: China, Xingxiang V6E (Chinese-market reference; EU-spec export build kept separate)

Verification method: Cross-check + official export record; battery-year and market scope isolated

Editorial standard: Researched and written from the sources listed above (desk research; no first-hand driving, teardown or import is claimed). Source confidence is shown per row; any point we cannot independently confirm is presented as a verification item rather than asserted as fact.

#AutoBridge #VehicleExport #Farizon #V6E #ElectricCargoVan