# Used Chinese EV Pre-Export Inspection: Battery SOH Done Properly, Plus Accident, Flood and Odometer Checks

## SEO Metadata

SEO Title: Used Chinese EV Inspection: Battery SOH, Flood & Odometer Checks

Meta Description: How to inspect a used Chinese EV before export — what SOH actually means under battery standards, a metered charge test, cell-balance reading, accident/flood screening and odometer-vs-cycle cross-checks, with no invented legal threshold.

Suggested URL: /guides/used-chinese-ev-inspection/

H1: Inspecting a Used Chinese EV Before Export: What the Battery Numbers Mean and What to Reject

Primary Keyword: used Chinese EV inspection battery SOH checklist

Secondary Search Terms: EV battery SOH standard QC/T 743, GB/T 31484 cycle life, used EV cell voltage imbalance, EV flood damage inspection, odometer rollback battery cycle count

Internal Link Suggestions: /guides/verify-china-car-export-supplier/ ; /guides/chinese-ev-charging-standard-compatibility/ ; /guides/import-chinese-ev-to-russia-eac-ottc/

Image Suggestions: diagnostic-tool SOH and cell-voltage readout; metered 20–80% charge worksheet; battery undertray inspection points; service/claims record sample

ALT Suggestions: "Independent diagnostic report showing SOH and maximum cell-voltage difference"; "metered charge-energy test worksheet"; "EV battery pack undertray inspection points"

## Scope: an Inspection Manual, Not a New-vs-Used Debate

The decision to buy used is taken as given; this page covers the technical inspection before export. The economics of a used EV are dominated by the high-voltage battery: a clean body with a worn pack can be worth less than the replacement cost after shipping, so the inspection order of operations is different from an ICE car. A central aim is to be precise about what "SOH 80%" actually is — an engineering reference, not a universal customs line.

## What SOH Is — and What the 80% Figure Actually Means

State of Health (SOH) expresses a pack's current maximum usable capacity relative to its when-new rated capacity (capacity-based SOH = aged capacity ÷ rated capacity). It is essential to separate an engineering end-of-life convention from a legal import threshold, because they are routinely confused:

The ~80% figure originates in battery standards and warranty practice, not import law. China's automotive-industry standard QC/T 743 treats capacity falling to 80% of nominal as an end-of-life reference; GB/T 31484 specifies traction-battery cycle-life *test methods*; IEC 62660 (parts 1/2) standardises lithium-ion cell performance/life testing; and the newer recommended national standard GB/T 46991.1-2025 (developed under MIIT/SAC) addresses the *accuracy and durability of on-board SOH/SOC displays*. These are standards-body documents (VERIFIED as to what they define).

There is no single universal legal SOH threshold for importing a used EV. No regulation cited in this research sets "80%" as a blanket customs or type-approval gate that applies across the EU, ASEAN or anywhere else. Be explicit about what 80% is not: it is not a used-car import threshold, it is not a unified battery-retirement law, and it is not every manufacturer's warranty threshold (a given warranty may use a different figure). A destination may impose its own condition rule and a buyer may set a private commercial line — but each is a *different instrument*. The destination certification body / importer's own acceptance standard governs, and must be confirmed per country rather than assumed.

This distinction changes how you use the number: treat the ~80% engineering convention purely as a commercial/engineering reference for pricing and rejection, while separately confirming whether the destination sets any legal condition. No chemistry-specific "85% NMC line" is asserted here because no standard or manufacturer source captured for this article establishes it as a rule.

## Obtaining a Defensible SOH Number

Read the in-car/cluster health display, but do not stop there — on-board displays are estimates and the new GB/T 46991.1 display-accuracy standard applies to new production, not necessarily to older used units.

Commission an independent third-party diagnostic report that records SOH and the maximum cell-to-cell voltage difference: a large imbalance flags weak modules even when the headline SOH looks healthy.

Chinese brands do not share one OBD/diagnostic protocol and third-party report formats are not standardised, so keep the raw report (tool, software version, date, ambient temperature) as evidence. Brand-official battery-health/warranty criteria should be requested from the brand where available rather than guessed.

## Cross-Check With a Metered Live Charge Test (a Screening Cross-Check, Not an SOH Calculation)

A static SOH value can be misrepresented; a charge test exercises the pack. Charge from roughly 20% to 80% on a metered charger, recording start/end SOC, kWh delivered, the power curve and elapsed time, and compare delivered energy against the expected ~60% of usable pack capacity. Treat this strictly as a screening cross-check, not as a way to compute SOH. Charger-delivered kWh is *not* equal to energy stored in the cells: the reading is affected by charging/conversion losses, thermal-management draw, ambient and pack temperature, BMS SOC calibration, and the reserve/top-bottom buffer. It can therefore only flag a gross anomaly. A shortfall on this screen does not by itself prove genuine capacity loss (and a clean screen does not prove full health); the formal capacity/SOH figure still requires a qualified diagnostic method or manufacturer/professional test data. Inability to hold the expected charge rate is a reason to commission that deeper inspection. Run the screen alongside the charging-standard check (GB/T in China vs the destination connector — see the compatibility guide) so a sound pack is not failed by an incompatible inlet.

## The High-Voltage Triad Beyond SOH

Pack underside: remove/inspect the bottom shield for removal/reseal marks, scrapes, deformation or non-factory repair.

Motor and power electronics: scan fault codes, check warning lamps and verify drive/regen behaviour on a road test.

HV cabling and connectors: look for corrosion, retermination or non-factory joins.

## Accident, Flood and Fire Screening

Pull maintenance records and insurance-claim (出险) records; structural sheet-metal repair or any flood-water claim marker is grounds to reject.

Flooded EVs hide pack degradation and harness corrosion: inspect under-dash connectors, seat rails, fuse boxes and the battery enclosure for water lines/corrosion.

Check panel gaps, paint thickness and fastener witness marks; treat any sign the battery pack was opened outside a qualified facility as a stop signal.

## Odometer vs Charge Cycles

A clocked EV shows an odometer inconsistent with battery cycle count and wear. Where diagnostics expose cycle count, compare it with displayed mileage and physical wear (seat, steering, pedals). Low displayed mileage paired with high cycle counts or aged components is a rollback warning that an ICE odometer check alone would miss.

## Paperwork and the Destination Rule

Keep the independent SOH/imbalance report, the metered charge record and service/claims history in the export dossier. Separately — and this is the correction to any blanket "you need 80% to import" claim — obtain the destination's actual used-EV condition/type rule in writing from its certification authority or your clearance agent. China's own used-car export qualification/age rules are set by MOFCOM and updated (see the supplier-vetting and fleet-procurement guides); use the current-year position rather than an older blog figure.

## What AutoBridge Adds Beyond a Generic Checklist

Public checklists repeat "read SOH and reject below 80%." This guide instead recommends: (1) separating the engineering 80% convention from the destination's actual legal rule, so a buyer neither walks away from a compliant car nor ships a non-compliant one on a myth; (2) pairing a metered-energy screening cross-check with a cell-imbalance reading rather than relying on a dashboard screenshot; and (3) keeping the inspection note tied to the VIN and export dossier used for certification, so the tested car is traceable to the shipped car.

## Hard Rejection Rules

Flood/fire marker in claims records, or corrosion/water lines in the pack or HV harness.

Structural/accident repair to high-voltage areas, or evidence the pack was opened outside a qualified facility.

Odometer inconsistent with cycle count and wear, with no credible explanation.

Seller refuses a metered charge test or an independent diagnostic report.

Capacity/imbalance that fails your own documented acceptance line (set from brand warranty + destination rule + commercial margin) — not a mythical universal 80% legal gate.

## Acceptance Test List

Independent report: SOH and max cell-voltage imbalance, with tool/version/date recorded.

Metered 20→80% charge test (SOC, kWh, power curve, time).

Battery undertray and HV connectors inspected for removal/corrosion.

Service + insurance-claim history pulled; collision/flood/fire markers checked.

Odometer reconciled with charge cycles and physical wear.

Destination used-EV rule confirmed in writing and distinguished from the engineering 80% line.

Charging-interface compatibility (GB/T vs destination) verified.

## Frequently Asked Questions

Is 80% SOH a legal import requirement? No. Roughly 80% is an engineering/warranty end-of-life reference (e.g., QC/T 743; tested per GB/T 31484 / IEC 62660); it is not a used-car import threshold, not a unified retirement law and not every manufacturer's warranty line. There is no universal 80% customs gate, so confirm the destination's own rule.

Why do people quote 80% then? It comes from battery standards and warranty practice as an end-of-life/commercial reference; it is useful for pricing and rejection, but it is not import law, and no chemistry-specific 85% rule is asserted here.

Can I calculate SOH from the charger's kWh between 20% and 80%? Not directly. Delivered energy is distorted by charging losses, thermal draw, temperature, BMS calibration and buffer; it is a screening cross-check that can reveal gross anomalies, with formal SOH left to a qualified diagnostic.

How should SOH be evidenced? An independent report showing SOH and cell-voltage imbalance, cross-checked by a metered 20–80% charge test, kept with tool/version/date.

Why is flood damage especially dangerous in an EV? It can degrade the pack and corrode hidden harnesses/connectors, causing safety and reliability failures after export.

How is odometer fraud spotted on an EV? Compare displayed mileage with battery cycle count and physical wear; low odometer with high cycles is a warning.

## Image Record

IMAGE_ASSET_PATH: none secured in repository

ORIGINAL_IMAGE_URL: not captured

SOURCE_PAGE: not captured

SOURCE_FILE_PAGE: not applicable — no candidate media file identified (no licence to assert)

RIGHTS_HOLDER: unconfirmed

LICENSE_OR_USAGE_BASIS: none secured — no third-party image may be published until rights are cleared

CHECKED_DATE: 2026-09-06

MODEL_TOPIC_MATCH: must match the exact model/version (or the guide topic) and reference market above

IMAGE_SCOPE_NOTE: match the exact model family/topic only; must not imply a specific trim/model-year, real VIN, in-person inspection or actual transaction

IMAGE_RIGHTS_STATUS: FAIL (no licensed asset captured; a placeholder or “retain old image” note is not accepted)

BLOCK_REASON: No reusable image could be secured: Wikimedia Commons/Flickr are unreachable from the research environment, stock libraries require authenticated API/licence access, and an OEM webpage image is NOT a commercial reuse grant; no AutoBridge-owned photo exists. Kept FAIL rather than asserted.

ALT by language:

EN: AutoBridge export-buyer reference — Used Chinese EV pre-export inspection, vehicle-export procurement guide

FR: Référence AutoBridge pour acheteurs export — Used Chinese EV pre-export inspection, guide d’achat à l’export automobile

DE: AutoBridge-Referenz für Exportkäufer — Used Chinese EV pre-export inspection, Leitfaden für Fahrzeugexport-Einkauf

ES: Referencia AutoBridge para compradores de exportación — Used Chinese EV pre-export inspection, guía de compras para exportación de vehículos

PT: Referência AutoBridge para compradores de exportação — Used Chinese EV pre-export inspection, guia de compras para exportação de veículos

JA: AutoBridge 輸出バイヤー向けリファレンス｜Used Chinese EV pre-export inspection, 自動車輸出 調達ガイド

KO: AutoBridge 수출 바이어 참고 자료｜Used Chinese EV pre-export inspection, 자동차 수출 조달 가이드

VI: Tài liệu tham khảo AutoBridge cho người mua xuất khẩu — Used Chinese EV pre-export inspection, hướng dẫn thu mua xuất khẩu xe

TH: เอกสารอ้างอิง AutoBridge สำหรับผู้ซื้อเพื่อการส่งออก — Used Chinese EV pre-export inspection, คู่มือจัดซื้อเพื่อการส่งออกยานยนต์

ID: Referensi AutoBridge untuk pembeli ekspor — Used Chinese EV pre-export inspection, panduan pengadaan ekspor kendaraan

AR: مرجع AutoBridge لمشتري التصدير — Used Chinese EV pre-export inspection, دليل مشتريات تصدير المركبات

ZH: AutoBridge 出口采购参考｜Used Chinese EV pre-export inspection, 汽车出口采购指南

## Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| GB/T 31484 traction-battery cycle-life requirements/test methods | Chinese national standard (**standards body**) | CN | https://www.chinesestandard.net/PDF.aspx/GBT31484-2015 | 2026-09-03 | **VERIFIED** | Capacity/cycle-life test framework; initial-capacity requirements |
| IEC 62660-1/2 lithium-ion cell performance/life testing | IEC (international **standards body**) | Global | https://www.iec.ch/ (IEC 62660 series) | 2026-09-03 | **VERIFIED** | Standardised cell performance/life test basis for SOH |
| GB/T 46991.1-2025 on-board SOH/SOC display accuracy & durability (MIIT/SAC) | Chinese recommended national standard (**standards body**) | CN | reported via standards coverage; primary on SAC/MIIT channels | 2026-09-03 | CROSS_CHECKED | On-board health-display accuracy is separately standardised (not a legal import line) |
| SOH formulations and the QC/T 743 80% end-of-life convention | LNC Batteries technical explainer (industry) | Global | https://lnclibattery.com/blog/evaluation-of-the-health-status-soh-of-lithium-ion-batteries/ | 2026-09-03 | SINGLE_SOURCE | Capacity-based SOH formula; 80% as industry end-of-life reference |
| Core points for buying used NEVs / charge-test & screening methods | Yiche, Dongchedi (auto media; method references) | CN | https://hao.m.yiche.com/wenzhang/107776270/ | 2026-09-03 | SINGLE_SOURCE | Inspection method, charge test, accident/flood/odometer practice |
| 懂车帝 二手车电池检测内容 | 懂车帝（字节跳动） | CN | https://www.iesdouyin.com/share/video/7618925618490849457 | 2026-09-02 | SINGLE_SOURCE | 20%-80% 充电验证衰减方法 |
| 懂车帝 二手电车三招排除事故/泡水/调表 | 懂车帝（视频） | CN | https://www.iesdouyin.com/share/video/7678314679869430004 | 2026-09-02 | SINGLE_SOURCE | 事故/泡水/调表排查方法、电池包护板拆装痕迹 |
| Jingsuncar — 2026 二手新能源出口指南 | Jingsuncar（行业站） | CN | https://www.jingsuncar.com/news/2026-must-see-guide-for-buying-used-new-energy-85493284.html | 2026-09-02 | SINGLE_SOURCE | 出口 SOH≥80% 认证门槛（EU/东盟） |

*Confidence note: what the battery standards define (SOH measurement, cycle testing, the 80% engineering convention) is VERIFIED/CROSS_CHECKED against standards bodies. The earlier "SOH ≥80% required to clear EU/ASEAN certification" claim had no official source and has been removed: no universal legal SOH import threshold exists, and the destination authority's rule must be obtained per country. Inspection-method videos are used only as method references.*

## Editorial Review

Author / reviewer: [AutoBridge Export Editorial Team](/authors/) · method per our [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-05

Reference market: Global (China used-EV export side)

Verification method: Standards-body basis for SOH concepts; media used only for inspection method; engineering line kept distinct from any destination legal rule

Editorial standard: Researched and written from the sources listed above (desk research; no first-hand driving, teardown or import is claimed). Source confidence is shown per row; any point we cannot independently confirm is presented as a verification item rather than asserted as fact.

#AutoBridge #UsedEVInspection #BatterySOH #ExportProcurement #PrePurchaseCheck