# Loading Cars Into a 40HQ Container: Counts That Depend on a Load Plan, and Lashing That Holds at Sea

## SEO Metadata

SEO Title: How Many Cars Fit in a 40HQ? Container Loading & Lashing Guide

Meta Description: A practical 40HQ vehicle-loading guide — typical 2-SUV no-rack loading vs 4-vehicle double-rack systems, ~8 straps per car on tyres, loading sequence and a five-point stowage check before seal.

Suggested URL: /guides/container-vehicle-loading-lashing/

H1: Container Vehicle Loading and Lashing: Counts, Racks, Sequence and Pre-Seal Checks

Primary Keyword: how many cars fit 40hq container loading lashing

Secondary Search Terms: 40HQ two SUV, double deck car rack container, vehicle lashing straps wheel tie down, RoRo to container conversion, container car loading sequence

Internal Link Suggestions: /guides/vehicle-pre-shipment-inspection-psi/ ; /guides/ev-shipping-un3171-imdg-compliance/ ; /vehicles/changan-cs75-plus/

Image Suggestions: side-view stowage diagram; wheel lashing points; pull-out double rack; top+bottom loading order

ALT Suggestions: "40HQ two SUV side stowage"; "wheel tyre lashing eight straps"

## The Question Everyone Asks First

"How many cars in a 40HQ?" has no universal answer — it is a function of vehicle dimensions, mirror-folding, rack system and the carrier's approved stowage plan. This guide gives realistic bands and the method; it deliberately does not promise a fixed count. Confirm every shipment against an actual measured load plan.

## Realistic Loading Bands (from documented practice)

No rack, conventional: a 40HQ typically carries two mid/large SUVs (examples cited are ~4.9 m class such as ID.6 / Range-Rover / Prado / Li L7-scale vehicles) (single-source forwarder diagram).

With a sliding pull-out double-deck rack: an "upper 2 + lower 2" = four SUVs arrangement was reported in one Yantai-port "RoRo-to-container" trial (single-source).

Sedans / China-Europe rail: three vehicles per box appears in China-Europe rail/sedan practice reporting (single-source).

These are reference points, not entitlements: a high-roof van, a pickup or a long executive car changes the count, and rack loading needs certified equipment and an approved plan.

## Lashing: Fix Through the Tyres, Not the Body

Documented practice for in-box securing:

About eight straps per vehicle, predominantly a wheel/tyre tie-down method with multi-directional (five-direction) load paths.

Straps act on the tyres/wheel points, not on body panels, paint or suspension components — lashing to suspension or chassis in the wrong place can damage the car and still move under sea motion.

Use anti-slip mats/chocks and timber blocking, set correct strap tension, and protect any contact point.

The objective is that under vessel rolling/pitching the car cannot shift, rotate or contact the box wall or the second vehicle.

## Loading Sequence (where rack work goes wrong)

For rack/double-deck loads, documented port practice is:

Load the racked vehicles top-first / in the planned order, using a forklift to assist push-and-reposition;

A double-door (both-end) container allows work from both ends simultaneously, shortening dwell;

Secure each unit progressively — never leave a vehicle unsecured while manoeuvring the next.

Sequence errors (loading the lower car first and blocking rack access) are the most avoidable delay in four-car loads.

## The Five-Point Pre-Seal Stowage Check

Adapted from documented container-loading method:

Goods vs packing list — VINs, quantities and condition match; PSI already completed (see PSI guide).

Loading order — weight distribution and rack positions per plan.

Centre of gravity / clearance / height — no contact with roof, walls or doors; even side-to-side weight.

Securing — straps, chocks, timber, anti-slip all in place and tensioned; nothing on paint/suspension.

Door clearance — enough margin to close and seal; record photos of the stow before sealing.

## EV-Specific Note

An EV/hybrid inside a container still carries a traction battery: follow the ocean dangerous-goods documentation and any carrier SOC/state-of-charge instruction. under IMDG Amendment 42-24 (mandatory from 2026-01-01) a lithium-ion-battery-powered vehicle is classified UN3556, not the legacy generic UN3171; apply documentation/SOC details per the current EV shipping Fact Sheet and the carrier (see the EV ocean-compliance guide) Disconnect/prepare the 12 V system per the carrier and do not assume an internal-combustion stowage SOP covers an EV.

## What This Guide Recommends You Lock Before Booking

A measured load plan for the exact models (length/width/height, mirrors folded), not a generic "two or four" promise.

Certified rack equipment and an approved four-car plan if using double deck.

Per-car strap count and wheel-point method agreed with the loading warehouse.

Photo evidence of stow and lashing before seal, linked to VINs.

EV dangerous-goods paperwork and carrier SOC rules where relevant.

Port, route, carrier and terminal acceptance in writing: name the actual load port and terminal (the documented trials above are China-port references such as Yantai, not a universal standard), the ocean carrier and its container operator, and whether that terminal accepts double-deck car racks and in-rack lashing on the booked sailing. Acceptance conditions and any effective-date change differ by terminal and carrier, so record the date each condition was confirmed against the CTU Code securing principles rather than assuming one warehouse's method is accepted everywhere.

## Frequently Asked Questions

Can a 40HQ always take four SUVs? No — four needs a certified pull-out double rack and an approved plan; conventional no-rack loading of mid/large SUVs is typically two.

Where should straps attach? To the tyres/wheel tie-down points — about eight per car — not to bodywork or suspension.

How do sedans differ? Sedans and China-Europe rail loads commonly achieve three per box; dimensions still decide.

Why load the rack top-first? The upper vehicles must be positioned before the lower ones block rack access; forklift assistance repositions them safely.

What is checked before sealing the doors? Goods/list match, order, centre of gravity and clearance, full securing, and door margin — with photos.

## Image Record

IMAGE_ASSET_PATH: none secured in repository

ORIGINAL_IMAGE_URL: not captured

SOURCE_PAGE: not captured

SOURCE_FILE_PAGE: not applicable — no candidate media file identified (no licence to assert)

RIGHTS_HOLDER: unconfirmed

LICENSE_OR_USAGE_BASIS: none secured — no third-party image may be published until rights are cleared

CHECKED_DATE: 2026-09-06

MODEL_TOPIC_MATCH: must match the exact model/version (or the guide topic) and reference market above

IMAGE_SCOPE_NOTE: match the exact model family/topic only; must not imply a specific trim/model-year, real VIN, in-person inspection or actual transaction

IMAGE_RIGHTS_STATUS: FAIL (no licensed asset captured; a placeholder or “retain old image” note is not accepted)

BLOCK_REASON: No reusable image could be secured: Wikimedia Commons/Flickr are unreachable from the research environment, stock libraries require authenticated API/licence access, and an OEM webpage image is NOT a commercial reuse grant; no AutoBridge-owned photo exists. Kept FAIL rather than asserted.

ALT by language:

EN: AutoBridge export-buyer reference — Loading cars into a 40HQ container, vehicle-export procurement guide

FR: Référence AutoBridge pour acheteurs export — Loading cars into a 40HQ container, guide d’achat à l’export automobile

DE: AutoBridge-Referenz für Exportkäufer — Loading cars into a 40HQ container, Leitfaden für Fahrzeugexport-Einkauf

ES: Referencia AutoBridge para compradores de exportación — Loading cars into a 40HQ container, guía de compras para exportación de vehículos

PT: Referência AutoBridge para compradores de exportação — Loading cars into a 40HQ container, guia de compras para exportação de veículos

JA: AutoBridge 輸出バイヤー向けリファレンス｜Loading cars into a 40HQ container, 自動車輸出 調達ガイド

KO: AutoBridge 수출 바이어 참고 자료｜Loading cars into a 40HQ container, 자동차 수출 조달 가이드

VI: Tài liệu tham khảo AutoBridge cho người mua xuất khẩu — Loading cars into a 40HQ container, hướng dẫn thu mua xuất khẩu xe

TH: เอกสารอ้างอิง AutoBridge สำหรับผู้ซื้อเพื่อการส่งออก — Loading cars into a 40HQ container, คู่มือจัดซื้อเพื่อการส่งออกยานยนต์

ID: Referensi AutoBridge untuk pembeli ekspor — Loading cars into a 40HQ container, panduan pengadaan ekspor kendaraan

AR: مرجع AutoBridge لمشتري التصدير — Loading cars into a 40HQ container, دليل مشتريات تصدير المركبات

ZH: AutoBridge 出口采购参考｜Loading cars into a 40HQ container, 汽车出口采购指南

## Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| Vehicle stowage side-view (2/box, 8 straps) | Freight-forwarder material | Global | https://m.11467.com/product/d42473652.htm | 2026-09-03 | SINGLE_SOURCE | 2-SUV no-rack; ~8 straps; wheel method (one source) |
| Yantai port first 4-SUV "RoRo-to-container" trial | Qilu Evening News / Toutiao | CN | http://m.toutiao.com/group/7443978035637846554/ | 2026-09-03 | SINGLE_SOURCE | Upper2+lower2 rack; loading sequence (one reported trial) |
| China-Europe rail car loading (3/box) | Harbin Railway / Toutiao | CN | http://m.toutiao.com/group/6748371578787987979/ | 2026-09-03 | SINGLE_SOURCE | Three-per-box sedan/rail reference (one source) |
| Export container five-step loading method | NetEase | Global | https://www.163.com/dy/article/L42056IP05568MR2.html | 2026-09-03 | SINGLE_SOURCE | Five-point stowage check (one source) |

*Scope note: each loading figure comes from a single, distinct source (all SINGLE_SOURCE) — no fact is corroborated by two independent sources, so none is labelled CROSS_CHECKED. Counts are documented-practice references, not guaranteed stowage; the exact number always depends on measured dimensions and an approved load plan, and carrier/terminal rules must be confirmed at booking.*

| IMO/ILO/UNECE Code of Practice for Packing Cargo Transport Units (CTU Code) | International Maritime Organization (IMO) | INT'L | https://www.imo.org/en/OurWork/Safety/Pages/CTU-Code.aspx | 2026-09-05 | VERIFIED | Cargo distribution, lashing and securing principles for CTUs including containers |
| --- | --- | --- | --- | --- | --- | --- |
| Containers / equipment reference | World Shipping Council | INT'L | https://www.worldshipping.org/ | 2026-09-05 | VERIFIED | Standard freight-container and liner-equipment context for load planning |

## Editorial Review

Author / reviewer: [AutoBridge Export Editorial Team](/authors/) · method per our [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-05

Reference market / scope: Global logistics operation (method, not a fixed-count promise)

Verification method: operational guidance, no universal count asserted

Editorial standard: Researched and written from the sources listed above (desk research; no first-hand driving, teardown or import is claimed). Source confidence is shown per row; any point we cannot independently confirm is presented as a verification item rather than asserted as fact.

#AutoBridge #ContainerLoading #40HQ #VehicleLashing #CTUCode