# Geely Galaxy L6 EM-i (2025) — Chinese-Market Specs and Export Verification

## SEO Metadata

SEO Title: Geely Galaxy L6 EM-i PHEV 2025: Chinese-Market Specs & Export Buyer Guide

Meta Description: Chinese-market Geely Galaxy L6 EM-i plug-in hybrid: verified dimensions, EM-i powertrain, two battery/range lines, and what an importer must confirm by VIN.

H1: Geely Galaxy L6 EM-i (2025) — Chinese-Market Specs and Export Verification

Primary Keyword: Geely / 吉利银河 Galaxy L6 / 银河L6 specs export

Secondary Search Terms: Geely Galaxy L6 export; Galaxy L6 EM-i PHEV specs; Chinese PHEV sedan import; EM-i single-speed DHT; CLTC vs WLTP range

Suggested URL: /vehicles/geely-galaxy-l6/

Search Intent: 了解 Geely / 吉利银河 Galaxy L6 / 银河L6 中国版规格、配置与出口适配性（车型参数页）

Internal Link Suggestions: /vehicles/geely-galaxy-l6/, /guides/incoterms-2020-selection-guide/, /guides/export-payment-methods-t-t-lc-risk/

Schema Scope: Article + Vehicle (no Product/Offer/Review)

## Buyer context and model identity

The Geely Galaxy L6 EM-i (2025) is a compact plug-in hybrid sedan built for the Chinese market. This page uses the 2025 EM-i facelift line only; the older Thor/8848 three-speed DHT versions are a different powertrain generation and must not be mixed with the EM-i single-speed cars described here. Reference market is China, and every figure below is a Chinese-market reference specification, not a global or export homologation value.

The EM-i line is front-wheel drive and pairs a 1.5 L naturally aspirated petrol engine with a single front motor through a one-speed dedicated hybrid transmission. Two battery/range lines are sold in China; an importer should treat them as distinct orderable configurations rather than one "L6" with a single range number.

## Variant and powertrain lines (Chinese market)

Architecture: FF (front-engine/front-drive), 4-door 5-seat sedan; body 4782 / 1875 / 1489 mm, wheelbase 2752 mm (cross-checked).

Engine: BHE15-BFN 1.5 L naturally aspirated, 1499 mL, 82 (112Ps) kW / 136 N·m (single-source ZOL parameter table).

Motor: 160 kW / 262 N·m (motor power cross-checked; torque single-source).

Transmission: 1挡 DHT（EM-i 单挡混动专用变速） — a single-speed DHT, which is the defining EM-i feature versus the older multi-speed hybrid.

Battery lines: 8.5 / 19.09 kWh. Battery cell supplier is not confirmed by Geely and is intentionally not stated.

Electric range: 60 / 140（对应8.5/19.09kWh） km CLTC for the two lines respectively; on WLTC the captured reference is 48 / 115 km. CLTC and WLTC are different cycles — never quote a CLTC number as a WLTC/NEDC export figure.

Combined/performance: combined CLTC range up to 最高 2000 km; 0–100 km/h 7.4 s; top speed 180 km/h; fuel tank 60 L; drag coefficient 0.26.

## Verified specification table (Chinese-market reference)

| Specification | Value | Unit | Market | Confidence | Source note |
| --- | --- | --- | --- | --- | --- |
| length width height | 4782 / 1875 / 1489 | mm | CHINA | CROSS_CHECKED | ZOL+搜狐双源一致 |
| wheelbase | 2752 | mm | CHINA | CROSS_CHECKED | ZOL+搜狐双源一致 |
| body style | 4门5座三厢轿车 | - | CHINA | SINGLE_SOURCE | 搜狐车型库 |
| engine model | BHE15-BFN 1.5L 自然吸气 | - | CHINA | SINGLE_SOURCE | ZOL参数表 |
| engine displacement | 1499 | mL | CHINA | SINGLE_SOURCE | ZOL参数表 |
| engine power | 82 (112Ps) | kW | CHINA | SINGLE_SOURCE | ZOL参数表 |
| engine torque | 136 | N·m | CHINA | SINGLE_SOURCE | ZOL参数表 |
| motor power | 160 | kW | CHINA | CROSS_CHECKED | ZOL+太平洋 |
| motor torque | 262 | N·m | CHINA | SINGLE_SOURCE | ZOL参数表 |
| transmission | 1挡 DHT（EM-i 单挡混动专用变速） | - | CHINA | SINGLE_SOURCE | 搜狐车型库 |
| battery capacity lines | 8.5 / 19.09 | kWh | CHINA | SINGLE_SOURCE | ZOL参数表 |
| ev range cltc | 60 / 140（对应8.5/19.09kWh） | km | CHINA | SINGLE_SOURCE | ZOL参数表 |
| ev range wltc | 48 / 115 | km | CHINA | SINGLE_SOURCE | 太平洋汽车 |
| combined range cltc | 最高 2000 | km | CHINA | SINGLE_SOURCE | 太平洋汽车 |
| top speed | 180 | km/h | CHINA | SINGLE_SOURCE | ZOL参数表 |
| accel 0 100 | 7.4 | s | CHINA | SINGLE_SOURCE | 太平洋汽车 |
| fuel tank | 60 | L | CHINA | SINGLE_SOURCE | 太平洋汽车 |
| drag coefficient | 0.26 | Cd | CHINA | SINGLE_SOURCE | 太平洋汽车 |

## Battery, range cycle and charging logic for buyers

The two battery lines drive both electric-only range and charging time, so an order should always name the battery capacity, not just the trim badge. Report range with its test cycle: the Chinese figures are CLTC, which reads higher than WLTP for the same hardware. Export-market powertrain output, battery chemistry certification and overseas range are not evidenced by an OEM export source here and stay unclaimed; request the destination-market homologation sheet instead of assuming the China EM-i calibration applies abroad. Infotainment chipset and overseas OTA availability are not confirmed and are excluded.

## Overseas buyer verification before payment

Lock the line: 8.5 kWh or 19.09 kWh; record the exact EM-i trim and obtain its VIN-level build line.

Cycle, not just number: ask which cycle any quoted range uses; reconcile CLTC vs the destination standard.

Powertrain generation: confirm EM-i single-speed DHT, not an older Thor hybrid.

Charging/connector: verify AC charge connector and any DC capability against the destination grid — do not assume China charging hardware.

Homologation & emissions: confirm PHEV type-approval, OBD/emissions stage and warranty territory for the destination country.

Quotation fields: destination country, quantity, exact battery line, LHD/RHD (China line is LHD), destination port and Incoterm. There is no AutoBridge export price list; request a current export quotation.

## FAQ

Is the Galaxy L6 EM-i the same car as the older Galaxy L6 hybrid? It is the same nameplate but a newer EM-i single-speed powertrain generation; do not merge specifications across them.

Why are two electric ranges quoted? They map to the 8.5 kWh and 19.09 kWh battery lines; both are CLTC Chinese-market figures.

Can I use the CLTC range in my market brochure? Not as a WLTP/NEDC figure — present it as Chinese-market CLTC and obtain the destination-cycle value.

Who makes the battery cells? Not OEM-confirmed in the captured sources, so this page does not name a supplier.

Does AutoBridge publish an export price? No — request a current quotation with destination, quantity, trim line, LHD/RHD, port and Incoterm.

## Sources & Verification

| Source | Organization | Market | Tier | Confidence | URL | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| 银河L6 系列参数 | 中关村在线 ZOL 产品库 | CN | T2 | CROSS_CHECKED | https://detail.zol.com.cn/series/2530/69637/param_10910168_0_1.html | 尺寸, 轴距, 动力, 电池, 续航 |
| 吉利银河L6 参数配置 | 搜狐汽车车型库 | CN | T2 | CROSS_CHECKED | https://db.auto.sohu.com/model_7163/config | 尺寸, 动力, 变速箱, 座位 |
| 银河L6 车型百科 | 太平洋汽车百科 | CN | T3 | CROSS_CHECKED | http://m.pcauto.com.cn/baike/1424735/2444736/ | 风阻, 油箱, 综合续航, 加速 |
| Geely official global site | Geely Auto Group 吉利 (OEM) | CN | T1 | VERIFIED | https://global.geely.com/ | brand/model-line identity and OEM authority |
| Ministry of Industry and Information Technology (MIIT) | MIIT 工业和信息化部 | CN | T1 | VERIFIED | https://www.miit.gov.cn/ | homologation/announcement authority; confirm exact model, masses and code by VIN/announcement |
| State Administration for Market Regulation (SAMR) | SAMR 国家市场监督管理总局 | CN | T1 | VERIFIED | https://www.samr.gov.cn/ | national standards, certification and market-regulation authority |

## Editorial Review

Author: AutoBridge Export Editorial Team · [authors](/authors/) · [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-05

Reference market: China (Chinese-market reference specification; not a global spec)

Verification method: Desk research against the sources below; Chinese-market reference specification only unless a separate export market is explicitly cited.

Transparency: AI-assisted drafting was used. This article is based on desk research and automated QA. No first-hand testing is claimed unless explicitly documented; confirm time-sensitive and destination-specific requirements before transacting.

## Image Record

IMAGE_ASSET_PATH: none secured in repository

ORIGINAL_IMAGE_URL: not captured

SOURCE_PAGE: not captured

SOURCE_FILE_PAGE: not applicable — no candidate media file identified (no licence to assert)

RIGHTS_HOLDER: unconfirmed

LICENSE_OR_USAGE_BASIS: none secured — OEM webpage presence is not a commercial reuse licence

CHECKED_DATE: 2026-09-06

MODEL_TOPIC_MATCH: exact Geely Galaxy L6 EM-i 2025

IMAGE_SCOPE_NOTE: image must match the exact model family/topic only; must not imply a specific trim, model year, real VIN, in-person inspection or an actual transaction

IMAGE_RIGHTS_STATUS: FAIL

BLOCK_REASON: No reusable image could be secured: Wikimedia Commons/Flickr are unreachable from the research environment, stock libraries require authenticated API/licence access, and an OEM webpage image is NOT a commercial reuse grant. No AutoBridge-owned photo exists. Record kept FAIL rather than asserted.

ALT (12 languages):

EN: Geely Galaxy L6 EM-i 2025, Chinese-market PHEV sedan export buyer reference

FR: Geely Galaxy L6 EM-i 2025, PHEV sedan du marché chinois, référence acheteur export

DE: Geely Galaxy L6 EM-i 2025, Chinesischer PHEV sedan, Referenz für Exportkäufer

ES: Geely Galaxy L6 EM-i 2025, PHEV sedan del mercado chino, referencia para comprador de exportación

PT: Geely Galaxy L6 EM-i 2025, PHEV sedan do mercado chinês, referência ao comprador de exportação

JA: Geely Galaxy L6 EM-i 2025, 中国市場PHEV sedan・輸出バイヤー向けリファレンス

KO: Geely Galaxy L6 EM-i 2025, 중국 시장 PHEV sedan, 수출 바이어 참고 자료

VI: Geely Galaxy L6 EM-i 2025, PHEV sedan thị trường Trung Quốc, tham chiếu cho người mua xuất khẩu

TH: Geely Galaxy L6 EM-i 2025, PHEV sedan ตลาดจีน เอกสารอ้างอิงสำหรับผู้ซื้อส่งออก

ID: Geely Galaxy L6 EM-i 2025, PHEV sedan pasar Tiongkok, referensi pembeli ekspor

AR: Geely Galaxy L6 EM-i 2025, PHEV sedan السوق الصيني، مرجع لمشتري التصدير

ZH: Geely Galaxy L6 EM-i 2025, 中国市场PHEV sedan·出口采购参考

#AutoBridge #ChinaCarExport #ExportProcurement #GeelyGalaxyL6 #PHEV