# JAC Kangling (康铃) Lingpao L6 Light Truck — Chassis Family, Announcement Builds and Export Sourcing Notes

## SEO Metadata

SEO Title: JAC Kangling Lingpao L6 Light Truck: Chassis, GVW & Driveline for Importers

Meta Description: JAC 1Ka Lingpao (Kangling) L6 light truck explained by chassis family — Anhui-Cummins 2.5 diesel power steps, 4T/5T rear axle, multiple MIIT announcement bodies, and how to pin the exact export build.

Suggested URL: /vehicles/jac-kangling-light-truck/

H1: JAC Kangling Lingpao L6: Matching Chassis, Announcement Model and Superstructure for an Export Order

Primary Keyword: JAC Kangling Lingpao L6 light truck specifications export

Secondary Search Terms: JAC Lingpao L6, HFC4DE3 Anhui Cummins 2.5, HFC1043 light truck, China 4.5t GVW diesel truck, JAC 1Ka light truck export, Kangling dropside truck

Internal Link Suggestions: /guides/commercial-truck-fleet-procurement-china/ ; /guides/import-chinese-truck-saudi-saso/ ; /vehicles/foton-aumark-refrigerated-truck/

Image Suggestions: side view of an L6 dropside; official chassis-frame and rear-axle layout; engine bay of the HFC4DE3 2.5 diesel; comparison of dropside/box/stake bodies; single-row 1920 mm cab

ALT Suggestions: "JAC Lingpao L6 4x2 diesel light dropside truck"; "Lingpao L6 180 mm chassis frame and 4T/5T rear axle"; "Anhui Cummins HFC4DE3 2.5-litre diesel engine"

## Why an L6 Is Ordered by Chassis Code, Not by a Model Name

A Chinese light commercial truck is not a fixed passenger-car specification: the Lingpao L6 (领跑L6, sold under JAC's 1Ka / 康铃 light-truck line) is a *chassis family* that JAC builds with several engines, two wheelbases, two rear-axle ratings and many factory or coachbuilt bodies (dropside 栏板, box 厢式, stake 仓栅, refrigerated and more). Each legal combination carries its own MIIT announcement model (公告型号) and its own curb weight / rated payload. "An L6" therefore says nothing precise until the announcement model, engine suffix and body are named. This page separates what JAC officially publishes about the platform from the single announcement build captured in the commercial-vehicle database, so an importer can order a defined truck rather than a name.

## The Platform, per JAC's Official Configuration Table

The following platform-level facts are taken from JAC Commercial Vehicle's official Lingpao L6 configuration table (cv.jac.com.cn, VERIFIED, checked 2026-09-03) and are stable across the family:

| Platform item | JAC-official value |
| --- | --- |
| Cab width | 1920 mm (standard-width / 33-mid cab), single-row and half-cab |
| Main wheelbase | 3365 mm (a 3600 mm long version exists on selected power trains) |
| Chassis frame section | 180 × 65 × 4.5 mm |
| Front axle | 2 T |
| Rear axle | 4 T / 5 T (a reinforced 10,000 N / 13,000 N option on some lines) |
| Final-drive ratios | 4.875 / 4.33 (4.11 on selected lines) |
| Fuel/power families offered | Zhiwei S25 (Q25A petrol-style), **Ankang 2.5 (Anhui-Cummins HFC4DE3 diesel)**, Ankang 3.0 cold-chain, YN30 gas / dual-fuel, Dekang 160B/C, DHE155 |

The diesel most relevant to export is the Ankang (安康) 2.5, using the Anhui-Cummins HFC4DE3 family. JAC's table lists three suffixes — HFC4DE3-2E5 / -2EA / -2E9 — at 111 / 117 / 125 kW, with torque steps of 360 and 458 N·m. This matters because resellers often quote a single "160 hp / 460 N·m" figure that does not match every suffix; the power step and torque must be read off the exact engine suffix and announcement certificate.

## One Representative Announcement Build (database, CROSS_CHECKED)

To show how a platform row becomes a concrete truck, the captured build is the dropside HFC1043P21K4C7S, 4×2, with figures from 360che/卡车之家 announcement data and a JAC-family cross-check (CROSS_CHECKED; captured 2026-09-02, re-checked against the JAC table 2026-09-03):

| Item | HFC1043P21K4C7S dropside (per announcement data) |
| --- | --- |
| Drive / wheelbase | 4×2 / 3365 mm |
| Overall L×W×H | 5.995 × 2.20 × 2.48 m |
| Cab | Single-row, 1920 mm, 3 seats |
| Engine | Anhui-Cummins HFC4DE3 series, 2.499 L inline-4 diesel, China VI |
| Gearbox | JAC ML645B 6-speed manual (a Fast 8-speed is used on certain other L6 variants) |
| Brakes | Full-air brakes with spring parking brake |
| Curb / rated payload / GVW | 2.565 t / 1.735 t / **4.495 t** |
| Front / rear axle | 2 T (1600 kg allowable) / 4 T (2895 kg group), ratio 4.875 |
| Tyres | 7.00R16LT 8PR, six wheels |
| Leaf springs | 4 / 5+6 |
| Dropside body | 4.18 × 2.10 × 0.40 m |

### Why the same "L6" can show different weights

Sister announcement builds carry different bodies and therefore different masses: the box (厢式, HFC5043XY… series) and stake (仓栅, HFC5043CCY… series) bodies are heavier, with curb weight around 2.7–2.8 t and rated payload near 1.5 t at the same ~4.495 t GVW. Never copy a dropside payload figure onto a box or reefer build — re-read it from that body's announcement certificate.

## Engine, Emission and Driveline: What to Pin on the Proforma

Engine suffix and power step: write the full HFC4DE3-2Ex code and its kW/N·m, not just "Cummins 2.5". Note China VI (国六) is a Chinese standard; it is not, by itself, proof of GCC, UNECE or destination emission acceptance — obtain the destination emission route separately.

Gearbox: confirm JAC 6-speed manual versus the Fast 8-speed used on some variants, and do not assume an automatic exists for the chosen power line.

Axle and ratio: choose 4 T vs 5 T rear axle and 4.875 vs 4.33 final drive against route grade and loaded GVW; the official table confirms both are factory options.

Brakes/tyres: air brakes with spring parking and 7.00R16LT 8PR ×6 on the captured build — confirm tyre availability and brake-service standards at destination.

## GVW, Body and Superstructure (the core of a light-truck buy)

At roughly 4.5 t GVW the L6 sits in the classic China light-truck class. For an export order, reconcile four numbers that Chinese paperwork combines: curb weight, rated payload, GVW and the destination-plated payload (which can differ after local body fitment). Decide dropside vs box vs stake vs reefer, record internal body dimensions and material, and state whether the superstructure is factory-fitted or locally coachbuilt. A refrigerated or insulated box materially raises curb weight and cuts net payload — see the Foton Aumark reefer page for how chassis, box and cooling unit are specified separately.

## What AutoBridge Adds Beyond the Parameter Sites

Public parameter sites list one configuration at a time and rarely expose that the numbers belong to *different announcement models*. Our editorial framework recommends: (1) mapping the quoted truck back to its MIIT announcement model and exact HFC4DE3 suffix, so a "160 hp" label is not silently treated as a 150 hp build; (2) preparing a VIN/announcement/engine-suffix consistency note before deposit; and (3) re-computing payload for the *actual* body being shipped rather than reusing a dropside figure. That version-identification step is what prevents the most common L6 order error.

## Overseas Buyer Verification Before Payment

Obtain the JAC factory specification sheet and the MIIT announcement certificate for the exact model code (e.g., HFC1043P21K4C7S) and reconcile every database figure to it.

Confirm engine suffix → kW/N·m, gearbox model, rear-axle rating and final-drive ratio in writing.

Confirm destination emission/fuel acceptance independently of China VI.

Reconcile GVW/payload/axle loads with local registration and operator-licensing classes for the body you ship.

Confirm LHD/RHD, and spare-parts supply for the Anhui-Cummins engine, the chosen gearbox and 7.00R16LT tyres.

Ask for the factory build sheet rather than a reseller spec card.

## Request a Current Export Quotation

No China domestic price is promoted to an export figure and none is invented here. Request a live quotation with Destination Country · Quantity · Announcement Model/Engine Suffix · Body Type · Axle/Ratio · LHD/RHD · Destination Port · Incoterm, and ask for the JAC official specification sheet and announcement certificate to be attached.

## Buyer FAQ

Is "Lingpao L6" one fixed specification? No — it is a chassis family with several engines, wheelbases, axle ratings and bodies; each combination has its own MIIT announcement model and weights.

What does the JAC official table confirm? 1920 mm cab, 3365 mm main wheelbase, 180×65×4.5 frame, 2 T front / 4–5 T rear axles, 4.875/4.33 ratios, and the HFC4DE3 Anhui-Cummins diesel at 111/117/125 kW steps.

Why do sources quote different horsepower for "the same" L6? Because the HFC4DE3 has three suffixes with different power/torque, and resellers sometimes quote one label across all of them; match the suffix on the announcement certificate.

What is the GVW and payload? The captured dropside build is 4.495 t GVW / 1.735 t payload; box and stake builds are heavier with lower payload — read each body's own certificate.

Is there an automatic? The captured diesel uses a JAC 6-speed manual; a Fast 8-speed appears on certain variants — confirm for the exact line rather than assuming.

## Image Record

IMAGE_ASSET_PATH: none secured in repository

ORIGINAL_IMAGE_URL: not captured

SOURCE_PAGE: not captured

SOURCE_FILE_PAGE: not applicable — no candidate media file identified (no licence to assert)

RIGHTS_HOLDER: unconfirmed

LICENSE_OR_USAGE_BASIS: none secured — no third-party image may be published until rights are cleared

CHECKED_DATE: 2026-09-06

MODEL_TOPIC_MATCH: must match the exact model/version (or the guide topic) and reference market above

IMAGE_SCOPE_NOTE: match the exact model family/topic only; must not imply a specific trim/model-year, real VIN, in-person inspection or actual transaction

IMAGE_RIGHTS_STATUS: FAIL (no licensed asset captured; a placeholder or “retain old image” note is not accepted)

BLOCK_REASON: No reusable image could be secured: Wikimedia Commons/Flickr are unreachable from the research environment, stock libraries require authenticated API/licence access, and an OEM webpage image is NOT a commercial reuse grant; no AutoBridge-owned photo exists. Kept FAIL rather than asserted.

ALT by language:

EN: AutoBridge export-buyer reference — JAC Kangling Lingpao L6, petrol/diesel Chinese-market vehicle / light truck

FR: Référence AutoBridge pour acheteurs export — JAC Kangling Lingpao L6, véhicule thermique (marché chinois) / camion léger

DE: AutoBridge-Referenz für Exportkäufer — JAC Kangling Lingpao L6, Verbrenner (chinesischer Markt) / Leicht-Lkw

ES: Referencia AutoBridge para compradores de exportación — JAC Kangling Lingpao L6, vehículo de combustión (mercado chino) / camión ligero

PT: Referência AutoBridge para compradores de exportação — JAC Kangling Lingpao L6, veículo a combustão (mercado chinês) / caminhão leve

JA: AutoBridge 輸出バイヤー向けリファレンス｜JAC Kangling Lingpao L6, 中国市場仕様 内燃機関車 / 小型トラック

KO: AutoBridge 수출 바이어 참고 자료｜JAC Kangling Lingpao L6, 중국 시장 내연기관 차량 / 경트럭

VI: Tài liệu tham khảo AutoBridge cho người mua xuất khẩu — JAC Kangling Lingpao L6, xe động cơ đốt trong (thị trường Trung Quốc) / xe tải nhẹ

TH: เอกสารอ้างอิง AutoBridge สำหรับผู้ซื้อเพื่อการส่งออก — JAC Kangling Lingpao L6, รถเครื่องยนต์สันดาป (ตลาดจีน) / รถบรรทุกขนาดเล็ก

ID: Referensi AutoBridge untuk pembeli ekspor — JAC Kangling Lingpao L6, kendaraan mesin pembakaran (pasar Tiongkok) / truk ringan

AR: مرجع AutoBridge لمشتري التصدير — JAC Kangling Lingpao L6, مركبة بمحرك احتراق (سوق الصين) / شاحنة خفيفة

ZH: AutoBridge 出口采购参考｜JAC Kangling Lingpao L6, 中国市场燃油车 / 轻卡

## Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| Lingpao L6 official configuration table | JAC Commercial Vehicle (JAC 江淮1卡, **manufacturer official**) | CN | https://cv.jac.com.cn/kllpL6/ | 2026-09-03 | **VERIFIED** | Cab 1920, WB 3365, frame 180×65×4.5, axles 2T/4–5T, ratios, HFC4DE3 power/torque steps, full power-family list |
| L6 HFC5043XY… box/announcement parameters | 360che (卡车之家, commercial DB) | CN | http://product.m.360che.com/m815/203872_param.html | 2026-09-02 | CROSS_CHECKED | Announcement model family, engine/gearbox, body weights |
| L6 representative dropside parameters | 360che (卡车之家) | CN | http://product.m.360che.com/m815/203878_param.html | 2026-09-02 | CROSS_CHECKED | HFC1043P21K4C7S dimensions, weights, axles, body, tyres |
| L6 Fast-8 / variant line | 360che (卡车之家) | CN | https://product.360che.com/m772/193093_index.html | 2026-09-03 | SINGLE_SOURCE | Alternative 8-speed gearbox, variant spread |

*Confidence per the AutoBridge standard: VERIFIED requires a manufacturer/government/standards body; JAC's own table meets that bar for platform facts. Detailed per-announcement weights from 360che are CROSS_CHECKED against the JAC family but still must be matched to the exact certificate. No price is stated; domestic list price is time-sensitive and never used as an export figure.*

| Road motor-vehicle manufacturers & products catalog — public query system | MIIT, China Ministry of Industry and Information Technology | CN | https://service.miit-eidc.org.cn/miitxxgk/gonggao_xxgk/index_ggcp.html | 2026-09-05 | VERIFIED | Verify the manufacturer, approved model and homologation (公告) catalog entry before ordering or export |
| --- | --- | --- | --- | --- | --- | --- |
| GB 16735-2019 Road vehicles — Vehicle identification number (VIN) requirements | SAMR national standards full-text platform | CN | https://openstd.samr.gov.cn/bzgk/std/newGbInfo?hcno=E2EBF667F8C032B1EDFD6DF9C1114E02 | 2026-09-05 | VERIFIED | VIN structure and nameplate data fields used in the buyer's VIN/nameplate verification |
| GB 7258 motor-vehicle safety technical conditions (GB 7258-2017 current; GB 7258-2026 applies from 2027-07-01) | SAMR, State Administration for Market Regulation | CN | https://www.samr.gov.cn/xw/sj/art/2026/art_9b2b3fa425e3461e8dc3bd270b410ceb.html | 2026-09-05 | VERIFIED | Baseline in-China vehicle safety technical conditions and the current/revision timeline |
| GB 18352.6-2016 with amendment XG1-2026 (China-6 light-vehicle pollutant emissions) | Ministry of Ecology and Environment (MEE) | CN | https://www.mee.gov.cn/xxgk2018/xxgk/xxgk01/202604/t20260430_1150676.html | 2026-09-05 | VERIFIED | China-6 light-vehicle emission stage and the 2026 amendment implementation dates |
| GB 19147 automobile diesel fuel national standard (low-sulphur basis) | SAMR national standards full-text platform | CN | https://openstd.samr.gov.cn/bzgk/std/newGbInfo?hcno=88F31AEECC7F7AE17C5A99496E532D2A | 2026-09-05 | VERIFIED | Diesel fuel grade and sulphur basis to match engine/emission design overseas |

## Editorial Review

Author / reviewer: [AutoBridge Export Editorial Team](/authors/) · editorial method per our [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-05

Reference market: China current-sale/announcement configuration (not a passenger model year)

Verification method: Manufacturer-official platform table + commercial-vehicle announcement database; per-build figures matched to announcement model; price intentionally omitted

Editorial standard: Researched and written from the sources listed above (desk research; no first-hand driving, teardown or import is claimed). Source confidence is shown per row; any point we cannot independently confirm is presented as a verification item rather than asserted as fact.

#AutoBridge #VehicleExport #JAC #Kangling #LightTruck