# Shacman Delong X3000 Dump Truck — 8×4 Tipper Specs and Export Configuration Notes

## SEO Metadata

SEO Title: Shacman X3000 Dump Truck Specs (8x4, GVW, Body) Export Guide

Meta Description: Shacman Delong X3000 tipper: 430 PS 8x4 reference build, Weichai engine, Fast 12-speed, 31 t GVW, 7.6 m rear-tip body, axles and export emission choices for fleet buyers.

Suggested URL: /vehicles/shacman-x3000-dump-truck/

H1: Shacman Delong X3000 Dump Truck: Chassis, Tipping Body and Export Build Choices

Primary Keyword: Shacman X3000 dump truck specifications export

Secondary Search Terms: Shacman X3000 8x4 tipper, Delong 430 dump truck, Weichai WP12 430, Fast 12-speed tipper, Chinese 31t dump truck export, Shacman 6x4 dump

Internal Link Suggestions: /vehicles/sinotruk-howo-t7h-tractor/ ; /guides/commercial-truck-fleet-procurement-china/ ; /guides/import-chinese-truck-saudi-saso/

Image Suggestions: 8x4 tipper side view; 7.6 m rear-tip body raised; MAN-type axles and suspension; Weichai WP12 engine; chassis frame detail

ALT Suggestions: "Shacman Delong X3000 8x4 dump truck, China reference build"; "X3000 7.6 metre rear-tipping body raised"; "Shacman X3000 tandem reduction axles"

## Role and Line-up

The Delong (德龙) X3000 is Shacman's volume heavy dump/tipper platform, exported widely across Africa, the Middle East and Russian-speaking markets. The reference China build is the 430 PS 8×4 with a 7.6 m body, announcement SX33105C406B; the family also spans 6×4/8×4 and roughly 375–550 PS, with China V/China VI and lower-emission (e.g., Euro II-class) builds coexisting for different destination markets. An official 400 PS 6×4 export version is confirmed by Shacman. Detailed China announcement figures come from 360che (single-source where marked); export ratings are official.

## Reference Build: 430 PS 8×4 (China)

| Item | Value | Evidence |
| --- | --- | --- |
| Announcement / drive | SX33105C406B, 8×4, wheelbase 1800+3975+1400 mm | 360che (single) |
| Overall L×W×H | 10.55 × 2.55 × 3.45 m | 360che (single) |
| Engine | Weichai WP12.430E50, 11.596 L inline-6 diesel | 360che (single) |
| Power / torque | 316 kW (430 PS) / 2060 N·m @ 1000–1400 rpm | 360che (single) |
| Gearbox | Fast (法士特) 12JSD200T-B **12-speed manual** (2 reverse); 460 PS China VI uses a 13-speed Fast | 360che (single) |
| Curb / rated payload / GVW | 15.5 t / 15.37 t / **31 t** | 360che (single) |
| Tipping body | **7.6 × 2.35 × 1.5 m, rear-tip (后翻)** | 360che (single) |
| Front axles | MAN-type 9.5 t, allowable 6500/6500 kg | 360che (single) |
| Rear bogie | 16 t MAN-type double-reduction, 18,000 kg group; ratio 5.262 | 360che (single) |
| Tyres / fuel | 12.00R20 ×12; 400 L aluminium tank | 360che (single) |

## Official Export Engine and Other Ratings

Official export 6×4 (VERIFIED): Weichai 11.6 L inline-6, 400 PS (294 kW) / 2000 N·m, China VI per the Shacman Group site; Shacman International lists 385/400/430 PS options and an 80 km/h governed top speed.

Higher China ratings include a 460 PS WP12.460E62 (338 kW/2200 N·m, China VI) and a 480 PS WP13.480E50 (353 kW, 12.54 L) — single-source database references for comparison only.

When ordering, match horsepower to the engine model suffix and emission stage: a 430 PS China-V engine is not the same certified product as a 400 PS export engine, even though outputs look close.

## Why Emission Version Is a Separate Decision

X3000 dumps are deliberately offered across China V, China VI and lower-spec (Euro II-class) calibrations because destination fuel quality and import rules differ so widely. A high-grade engine run on high-sulphur fuel can fail early; an over-specified emission system may be un-serviceable locally. Specify the emission level the destination actually requires and that its fuel supports — and obtain the matching engine/emission certificate rather than accepting a generic "X3000" description.

## What AutoBridge Adds Beyond a Truck Spec Card

Dump-truck quotes routinely blur the 400 PS official export engine and the 430 PS China-announcement build. This guide recommends fixing the exact engine code and emission stage to the VIN/announcement certificate, matching the axle ratio and 12.00R20 tyre/rim choice to the haul grade, and specifying the bed steel grade, volume and lift system in an inspectable annex — guarding against substitution of a weaker 8×4 build that still carries the X3000 name.

## Tipping and Chassis Checks for Buyers

Body and hydraulics: confirm body length (7.6 m reference), side/board height, steel grade, rear-tip vs side-tip, and the hydraulic hoist brand/capacity; bodies are often built to order.

Axles and final drive: the reference uses MAN-type double-reduction axles at 5.262 — suited to loaded/grade work; confirm ratio against route and tyre size (12.00R20, 12 tyres on 8×4).

Weights: re-confirm 31 t GVW and 15.37 t rated payload against destination axle-load and overload rules; body material/equipment changes curb weight.

Drive layout: choose 6×4 vs 8×4 by legal payload and road conditions; the 8×4 spreads load over four axles.

Gearbox: reference is a manual Fast 12-speed (13-speed on some China VI) — confirm transmission model and clutch spec; AMT/automatic is not assumed.

Confirm LHD/RHD, cab/sleeper, tank size and spare-parts chain for Weichai + Fast + MAN-type axles (a common, well-supported combination in many export markets).

## Request a Current Export Quotation

No authoritative price was captured and none is estimated. Enquire with Destination Country · Quantity · 6×4/8×4 · Horsepower & Engine Model · Emission Stage · Body Size/Tip Type · LHD/RHD · Destination Port · Incoterm, requesting the official Shacman build sheet and emission certificate.

## Buyer FAQ

What does the 8×4 430 reference include? A Weichai 11.596 L 430 PS engine, Fast 12-speed manual, 31 t GVW and a 7.6 × 2.35 × 1.5 m rear-tip body, per 360che announcement data.

Is there an official export version? Yes — Shacman confirms a 400 PS 6×4 (294 kW/2000 N·m, China VI) and lists 385/400/430 PS options internationally.

Why do emission versions vary so much? Markets differ in fuel sulphur content and import rules, so China V/VI and lower Euro-class calibrations coexist; match the engine to the destination.

What is the rated payload? Around 15.37 t for the reference 8×4 (single-source), subject to destination axle/GVW rules and final body build.

Manual or automatic? The captured builds use Fast manual transmissions (12/13-speed); confirm rather than assume an automatic.

## Image Record

IMAGE_ASSET_PATH: none secured in repository

ORIGINAL_IMAGE_URL: not captured

SOURCE_PAGE: not captured

SOURCE_FILE_PAGE: not applicable — no candidate media file identified (no licence to assert)

RIGHTS_HOLDER: unconfirmed

LICENSE_OR_USAGE_BASIS: none secured — no third-party image may be published until rights are cleared

CHECKED_DATE: 2026-09-06

MODEL_TOPIC_MATCH: must match the exact model/version (or the guide topic) and reference market above

IMAGE_SCOPE_NOTE: match the exact model family/topic only; must not imply a specific trim/model-year, real VIN, in-person inspection or actual transaction

IMAGE_RIGHTS_STATUS: FAIL (no licensed asset captured; a placeholder or “retain old image” note is not accepted)

BLOCK_REASON: No reusable image could be secured: Wikimedia Commons/Flickr are unreachable from the research environment, stock libraries require authenticated API/licence access, and an OEM webpage image is NOT a commercial reuse grant; no AutoBridge-owned photo exists. Kept FAIL rather than asserted.

ALT by language:

EN: AutoBridge export-buyer reference — Shacman Delong X3000, petrol/diesel Chinese-market vehicle / dump/tipper truck

FR: Référence AutoBridge pour acheteurs export — Shacman Delong X3000, véhicule thermique (marché chinois) / camion benne

DE: AutoBridge-Referenz für Exportkäufer — Shacman Delong X3000, Verbrenner (chinesischer Markt) / Kipper

ES: Referencia AutoBridge para compradores de exportación — Shacman Delong X3000, vehículo de combustión (mercado chino) / camión volqueta

PT: Referência AutoBridge para compradores de exportação — Shacman Delong X3000, veículo a combustão (mercado chinês) / caminhão basculante

JA: AutoBridge 輸出バイヤー向けリファレンス｜Shacman Delong X3000, 中国市場仕様 内燃機関車 / ダンプカー

KO: AutoBridge 수출 바이어 참고 자료｜Shacman Delong X3000, 중국 시장 내연기관 차량 / 덤프트럭

VI: Tài liệu tham khảo AutoBridge cho người mua xuất khẩu — Shacman Delong X3000, xe động cơ đốt trong (thị trường Trung Quốc) / xe ben

TH: เอกสารอ้างอิง AutoBridge สำหรับผู้ซื้อเพื่อการส่งออก — Shacman Delong X3000, รถเครื่องยนต์สันดาป (ตลาดจีน) / รถดั๊มพ์

ID: Referensi AutoBridge untuk pembeli ekspor — Shacman Delong X3000, kendaraan mesin pembakaran (pasar Tiongkok) / truk dump

AR: مرجع AutoBridge لمشتري التصدير — Shacman Delong X3000, مركبة بمحرك احتراق (سوق الصين) / شاحنة قلابة

ZH: AutoBridge 出口采购参考｜Shacman Delong X3000, 中国市场燃油车 / 自卸车

## Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| X3000 430 8×4 parameters | 360che (卡车之家) | CN | https://product.m.360che.com/m355/88939_param.html | 2026-09-02 | CROSS_CHECKED | Announcement, WP12.430, gearbox, weights, body, axles, tyres |
| X3000 6×4 dump official | SHACMAN Group (official) | CN/Export | https://www.shacman-group.com/products/shacman-x3000-6x4-dump-truck/ | 2026-09-02 | VERIFIED | 11.6 L 400 PS/294 kW/2000 N·m, China VI |
| X3000 multi-version compare | 360che (卡车之家) | CN | https://product.m.360che.com/m299/74922_param.html | 2026-09-02 | CROSS_CHECKED | 375–550 PS versions, China V/VI torque |
| X3000 international | Shacman International (official) | Export | https://www.shacmaninternational.cn/product-38.html | 2026-09-02 | VERIFIED | 80 km/h top speed, wheelbase, 385/400/430 options |

| Road motor-vehicle manufacturers & products catalog — public query system | MIIT, China Ministry of Industry and Information Technology | CN | https://service.miit-eidc.org.cn/miitxxgk/gonggao_xxgk/index_ggcp.html | 2026-09-05 | VERIFIED | Verify the manufacturer, approved model and homologation (公告) catalog entry before ordering or export |
| --- | --- | --- | --- | --- | --- | --- |
| GB 16735-2019 Road vehicles — Vehicle identification number (VIN) requirements | SAMR national standards full-text platform | CN | https://openstd.samr.gov.cn/bzgk/std/newGbInfo?hcno=E2EBF667F8C032B1EDFD6DF9C1114E02 | 2026-09-05 | VERIFIED | VIN structure and nameplate data fields used in the buyer's VIN/nameplate verification |
| GB 7258 motor-vehicle safety technical conditions (GB 7258-2017 current; GB 7258-2026 applies from 2027-07-01) | SAMR, State Administration for Market Regulation | CN | https://www.samr.gov.cn/xw/sj/art/2026/art_9b2b3fa425e3461e8dc3bd270b410ceb.html | 2026-09-05 | VERIFIED | Baseline in-China vehicle safety technical conditions and the current/revision timeline |
| GB 17691-2018 heavy-duty diesel vehicle pollutant emissions (China-VI) | Ministry of Ecology and Environment (MEE) | CN | https://www.mee.gov.cn/xxgk2018/xxgk/xxgk01/201807/t20180703_629590.html | 2026-09-05 | VERIFIED | China-VI compression-ignition (heavy diesel) emission stage for trucks/coaches |
| GB 19147 automobile diesel fuel national standard (low-sulphur basis) | SAMR national standards full-text platform | CN | https://openstd.samr.gov.cn/bzgk/std/newGbInfo?hcno=88F31AEECC7F7AE17C5A99496E532D2A | 2026-09-05 | VERIFIED | Diesel fuel grade and sulphur basis to match engine/emission design overseas |

*Confidence note (AutoBridge standard): the 400 PS export engine is VERIFIED on Shacman-official channels; the detailed 430 PS 8×4 announcement figures are SINGLE_SOURCE (360che). Price is UNVERIFIED and omitted. Confirm the final build on the Shacman International sheet.*

## Editorial Review

Author / reviewer: [AutoBridge Export Editorial Team](/authors/) · method per our [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-05

Reference market: China reference build + official export configurations

Verification method: Official Shacman channels plus an independent commercial database; single-source items labelled

Editorial standard: Researched and written from the sources listed above (desk research; no first-hand driving, teardown or import is claimed). Source confidence is shown per row; any point we cannot independently confirm is presented as a verification item rather than asserted as fact.

#AutoBridge #VehicleExport #Shacman #DelongX3000 #DumpTruck