# FAW Jiefang J6P Heavy Tractor: Engine, Gearbox, Axle and GCW Read as a Tractor-Spec Decision

## SEO Metadata

SEO Title: FAW Jiefang J6P Tractor Specs: Engine, GCW & Export Notes

Meta Description: Chinese-market FAW Jiefang J6P heavy tractor — Xichai CA6DM2 460 PS/2300 N·m China-VI diesel, 4×2/6×4/8×4 layouts, 10–16-speed gearboxes, ~40 t GCW class, and the export checks buyers need.

Suggested URL: /vehicles/faw-jiefang-j6p-tractor-specs/

H1: FAW Jiefang J6P (Chinese-Market): Heavy Tractor Powertrain, Chassis and Export Verification

Primary Keyword: faw jiefang j6p tractor specifications

Secondary Search Terms: Jiefang J6P 460, CA6DM2 engine, J6P 6x4 tractor GCW, China VI heavy truck export, J6P AMT gearbox, Xichai 11 litre diesel

Internal Link Suggestions: /vehicles/yutong-zk6122-coach-specs/ ; /vehicles/sany-concrete-mixer-truck-specs/ ; /guides/china-vehicle-export-license-customs-documents/

Image Suggestions: 6×4 tractor side; Xichai CA6DM2 engine; 12-speed gearbox; tandem drive axle; cab interior

ALT Suggestions: "FAW Jiefang J6P 6x4 heavy tractor"; "J6P Xichai CA6DM2 diesel engine"

## Buyer Context: Spec a Tractor by Driveline and GCW, Not by Badge

For a heavy haulage tractor the decisive fields are engine displacement and torque, gearbox, drive axles, GCW class, cab and chassis — not trim-level equipment. The J6P is FAW Jiefang's long-running heavy tractor series, offered in 4×2, 6×4 and 8×4 drive layouts. This page is a Chinese-market reference (REFERENCE MARKET: CHINA); every tractor is configured to a homologated model code, and export emission/axle ratios are order-specific.

## Engine (the anchor of a tractor quote)

| Item | J6P "460" China reference |
| --- | --- |
| Engine | Xichai **CA6DM2-46E6**, 11.05 L inline-6 diesel |
| Emission | China VI (国六) |
| Power | 338–341 kW (≈460 PS) |
| Peak torque | 2300 N·m @ 1000–1400 rpm |
| After-treatment | EGR + DOC + DPF + SCR (China VI / China VI-b) |

The engine model and its 2300 N·m torque are cross-checked across two Chinese commercial-vehicle sources. A higher-tier CA6DM3 13 L ≈560 PS ("3.0") with a reported ~150,000 km long-drain interval is single-source and must be confirmed on the exact code rather than assumed.

## Gearbox, Axles and GCW

Gearboxes: 10- to 16-speed manuals (example CA12TAX260M 12-speed), with AMT optional — specify the exact model and whether AMT is required.

Drive layouts: 4×2 / 6×4 / 8×4; line-haul export orders most commonly evaluate 6×4.

GCW class: the 6×4 sits in the approximately 40-tonne GCW class (single-source; confirm the rated GCW on the homologation certificate, as it changes with axle/tyre/suspension choice).

Confirm final-drive/axle ratio, fifth-wheel position, suspension (leaf/air), tyre size and fuel-tank capacity for the route's terrain and legal weights.

## Cab and Chassis

J6P offers multiple cab variants (day/sleeper, roof height) for long-haul vs regional work. Chassis and wheelbase differ with drive layout and intended semi-trailer match. Because tractor + trailer compatibility is market-specific (kingpin, fifth-wheel height, brake coupling), record these on the build sheet.

## Emission and Fuel Reality

China-VI diesels use EGR+DOC+DPF+SCR and require ultra-low-sulphur diesel and DEF (AdBlue). Before export, verify (a) the destination fuel sulphur level supports the after-treatment system, (b) whether China-VI is accepted or a different emission build is needed, and (c) DPF regeneration conditions in the operating climate. A China-VI tractor cannot simply be "derogated" without an engineering/approval basis.

## What Importers Should Verify Before Payment

Exact model code → engine (CA6DM2 vs CA6DM3), power, emission stage.

Drive layout and rated GCW on the certificate; axle ratio and suspension.

Gearbox model and AMT/manual, clutch and PTO needs.

Fuel sulphur / DEF / DPF compatibility and destination type approval.

Fifth-wheel/trailer coupling, LHD/RHD, cab spec, parts and service for Xichai engines in-market; PSI of VIN/engine number before shipment.

## Request a Current Export Quotation

No static export price and no China MSRP were captured; none is invented. Enquire with Destination Country · Model Code · Drive Layout · Engine/Emission · LHD/RHD · Destination Port · Incoterm.

## Frequently Asked Questions

What engine is in the J6P 460? The Xichai CA6DM2-46E6, an 11.05 L inline-6 China-VI diesel at about 460 PS and 2300 N·m.

What drive layouts exist? 4×2, 6×4 and 8×4; 6×4 is the common line-haul choice.

Is an automatic gearbox available? AMT is optional alongside 10–16-speed manuals; name the exact gearbox model.

What is the GCW? The 6×4 is in the ~40 t class, but rated GCW depends on axle/tyre/suspension — confirm on the certificate.

Can a China-VI tractor run on any diesel? It needs ultra-low-sulphur diesel plus DEF and proper DPF conditions; verify destination fuel and emission acceptance before ordering.

## Image Record

IMAGE_ASSET_PATH: none secured in repository

ORIGINAL_IMAGE_URL: not captured

SOURCE_PAGE: not captured

SOURCE_FILE_PAGE: not applicable — no candidate media file identified (no licence to assert)

RIGHTS_HOLDER: unconfirmed

LICENSE_OR_USAGE_BASIS: none secured — no third-party image may be published until rights are cleared

CHECKED_DATE: 2026-09-06

MODEL_TOPIC_MATCH: must match the exact model/version (or the guide topic) and reference market above

IMAGE_SCOPE_NOTE: match the exact model family/topic only; must not imply a specific trim/model-year, real VIN, in-person inspection or actual transaction

IMAGE_RIGHTS_STATUS: FAIL (no licensed asset captured; a placeholder or “retain old image” note is not accepted)

BLOCK_REASON: No reusable image could be secured: Wikimedia Commons/Flickr are unreachable from the research environment, stock libraries require authenticated API/licence access, and an OEM webpage image is NOT a commercial reuse grant; no AutoBridge-owned photo exists. Kept FAIL rather than asserted.

ALT by language:

EN: AutoBridge export-buyer reference — FAW Jiefang J6P, petrol/diesel Chinese-market vehicle / heavy tractor truck

FR: Référence AutoBridge pour acheteurs export — FAW Jiefang J6P, véhicule thermique (marché chinois) / tracteur routier lourd

DE: AutoBridge-Referenz für Exportkäufer — FAW Jiefang J6P, Verbrenner (chinesischer Markt) / schwere Sattelzugmaschine

ES: Referencia AutoBridge para compradores de exportación — FAW Jiefang J6P, vehículo de combustión (mercado chino) / cabeza tractora pesada

PT: Referência AutoBridge para compradores de exportação — FAW Jiefang J6P, veículo a combustão (mercado chinês) / cavalo mecânico pesado

JA: AutoBridge 輸出バイヤー向けリファレンス｜FAW Jiefang J6P, 中国市場仕様 内燃機関車 / 大型トラクターヘッド

KO: AutoBridge 수출 바이어 참고 자료｜FAW Jiefang J6P, 중국 시장 내연기관 차량 / 대형 트랙터 트럭

VI: Tài liệu tham khảo AutoBridge cho người mua xuất khẩu — FAW Jiefang J6P, xe động cơ đốt trong (thị trường Trung Quốc) / đầu kéo hạng nặng

TH: เอกสารอ้างอิง AutoBridge สำหรับผู้ซื้อเพื่อการส่งออก — FAW Jiefang J6P, รถเครื่องยนต์สันดาป (ตลาดจีน) / หัวลากบรรทุกหนัก

ID: Referensi AutoBridge untuk pembeli ekspor — FAW Jiefang J6P, kendaraan mesin pembakaran (pasar Tiongkok) / truk traktor berat

AR: مرجع AutoBridge لمشتري التصدير — FAW Jiefang J6P, مركبة بمحرك احتراق (سوق الصين) / رأس قاطرة ثقيل

ZH: AutoBridge 出口采购参考｜FAW Jiefang J6P, 中国市场燃油车 / 重型牵引车

## Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| New J6P 460 (CA6DM2-46E6 / 2300 N·m) | Truck Home / Toutiao | CN | http://m.toutiao.com/group/7109372278135095819/ | 2026-09-03 | CROSS_CHECKED | Engine model, power/torque, China VI |
| J6P comparison (CA6DM2-46E61, 341 kW) | Truck Home / Toutiao | CN | http://m.toutiao.com/group/7187639730950013444/ | 2026-09-03 | CROSS_CHECKED | 338–341 kW cross-check, drive layouts |
| J6P 3.0 (CA6DM3 13 L 560 PS) | Truck Home / Toutiao | CN | http://m.toutiao.com/group/6919343124628374029/ | 2026-09-03 | SINGLE_SOURCE | 13 L 560 PS, long-drain interval |
| Jiefang J6 model/drive/power lineage | Douyin Encyclopedia | CN | https://m.baike.com/wiki/%E8%A7%A3%E6%94%BEJ6/2354654 | 2026-09-03 | SINGLE_SOURCE | Layout/lineage background |
| Road motor-vehicle manufacturers & products catalog — public query system | MIIT, China Ministry of Industry and Information Technology | CN | https://service.miit-eidc.org.cn/miitxxgk/gonggao_xxgk/index_ggcp.html | 2026-09-05 | VERIFIED | Verify the manufacturer, approved model and homologation (公告) catalog entry before ordering or export |
| GB 16735-2019 Road vehicles — Vehicle identification number (VIN) requirements | SAMR national standards full-text platform | CN | https://openstd.samr.gov.cn/bzgk/std/newGbInfo?hcno=E2EBF667F8C032B1EDFD6DF9C1114E02 | 2026-09-05 | VERIFIED | VIN structure and nameplate data fields used in the buyer's VIN/nameplate verification |
| GB 7258 motor-vehicle safety technical conditions (GB 7258-2017 current; GB 7258-2026 applies from 2027-07-01) | SAMR, State Administration for Market Regulation | CN | https://www.samr.gov.cn/xw/sj/art/2026/art_9b2b3fa425e3461e8dc3bd270b410ceb.html | 2026-09-05 | VERIFIED | Baseline in-China vehicle safety technical conditions and the current/revision timeline |
| GB 17691-2018 heavy-duty diesel vehicle pollutant emissions (China-VI) | Ministry of Ecology and Environment (MEE) | CN | https://www.mee.gov.cn/xxgk2018/xxgk/xxgk01/201807/t20180703_629590.html | 2026-09-05 | VERIFIED | China-VI compression-ignition (heavy diesel) emission stage for trucks/coaches |
| GB 19147 automobile diesel fuel national standard (low-sulphur basis) | SAMR national standards full-text platform | CN | https://openstd.samr.gov.cn/bzgk/std/newGbInfo?hcno=88F31AEECC7F7AE17C5A99496E532D2A | 2026-09-05 | VERIFIED | Diesel fuel grade and sulphur basis to match engine/emission design overseas |

*Confidence note: no FAW primary spec sheet was captured; the 460 engine/torque are cross-checked (CROSS_CHECKED), while the 13 L 560 PS and ~40 t GCW are single-source and bound to the homologated model code. China-VI after-treatment compatibility is an engineering/approval check, not assumed.*

## Editorial Review

Author / reviewer: [AutoBridge Export Editorial Team](/authors/) · method per our [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-05

Reference market: China, J6P tractor (Chinese-market reference; order-specific driveline)

Verification method: Commercial-vehicle cross-check; single-source items and scope flagged

Editorial standard: Researched and written from the sources listed above (desk research; no first-hand driving, teardown or import is claimed). Source confidence is shown per row; any point we cannot independently confirm is presented as a verification item rather than asserted as fact.

#AutoBridge #VehicleExport #FAWJiefang #J6P #HeavyTractor