# Farizon Xingxiang V6E Electric Cargo Van: Battery-Year Lock, 6 m³ Load Bay and a Documented Türkiye Export

# SEO Metadata

SEO Title: Farizon V6E Electric Cargo Van Specs & Export Buyer Notes

Meta Description: Chinese-market Farizon (Geely) Xingxiang V6E electric cargo van — 60 kW/220 N·m motor, 6 m³ cargo volume, 35.2 kWh/260 km base vs newer 51 kWh build, 90 km/h limit, and a recorded EU-spec Türkiye shipment.

Suggested URL: /vehicles/farizon-xingxiang-v6e-specs/

H1: Farizon Xingxiang V6E (Chinese-Market): Electric Cargo Van Specs and Export Verification

Primary Keyword: farizon v6e electric cargo van specs

Secondary Search Terms: Xingxiang V6E range, Farizon electric delivery van, 6m3 electric van, V6E 35.2 kWh, Farizon Türkiye export, electric city logistics van

Internal Link Suggestions: /vehicles/saic-maxus-v90-specs/ ; /vehicles/byd-dolphin/ ; /guides/ev-shipping-un3171-imdg-compliance/

Image Suggestions: cargo bay with dimensions; side loading door; motor/battery layout; leaf-spring rear axle; EU-spec export batch

ALT Suggestions: "Farizon V6E six cubic metre electric cargo van"; "V6E battery grades and range"

# Buyer Context: Lock the Battery Year Before Anything Else

The Xingxiang V6E is Farizon's (Geely's commercial-vehicle brand) battery-electric urban delivery van. The single biggest spec risk is model year: the base build uses a 35.2 kWh pack while a newer build uses about 51 kWh, and range changes accordingly. A purchase order that says only "V6E" can be filled with either. This page covers the Chinese-market V6E (REFERENCE MARKET: CHINA) and keeps the base and newer builds separate.

# Driveline and Cargo (electric-van focus)

| Item | V6E China reference |
| --- | --- |
| Motor | Wolong-ZF TZ185XS022 permanent-magnet, 60 kW / 220 N·m |
| Top speed | 90 km/h (electronically limited) |
| Cargo volume | 6 m³ (cargo bay ~1.6 m long reference) |
| Rear suspension | Rigid axle with leaf springs (load-oriented) |
| Base battery/range | 35.2 kWh / 260 km working-condition range |
| Newer build | ≈51–51.4 kWh Xuanwu battery / 355–375 km (TIME_SENSITIVE — confirm current model) |

For a logistics van, buyers should weigh cargo volume, payload (confirm on the homologated code), the load-oriented leaf-spring rear axle and the 90 km/h limit against route speed needs — this is a city/last-mile vehicle, not a long-haul truck.

# Battery-Year Discipline

Base (this page's anchor): 35.2 kWh, ~260 km working-condition range (cross-checked).

Newer build: ~51 kWh Xuanwu pack, ~355–375 km (time-sensitive; availability and exact figures change).

Quote pack kWh and range and range cycle/condition together; never attach the newer 51 kWh number to a base VIN. Confirm AC/DC charging provision and charge time per build.

# Documented Export Record (fact, not an alias)

China Transport News (an industry news outlet) reports that in February 2024 an initial batch of EU-specification V6E vans was exported to Türkiye, described as Farizon's first entry into that market, in 6 m³ form. This is an export-delivery record: it confirms a European-standard build was shipped, but it does not make the Türkiye EU-spec van identical to the China 35.2/51 kWh builds — treat export configuration separately and request its spec sheet.

# What Importers Should Verify Before Payment

Battery build (35.2 vs ~51 kWh) and range cycle matched to VIN/model year.

Payload, GVW, cargo volume and rear-axle on the homologated code; leaf-spring ride vs cargo needs.

Charge interface (GB/T on China builds) vs destination standard; AC/DC provision.

EU-spec vs China-spec differences for any Türkiye/Europe-bound order (lighting, conformity, software).

90 km/h speed limit suitability; UN38.3/MSDS and under IMDG Amendment 42-24 (mandatory from 2026-01-01) a lithium-ion-battery-powered vehicle is classified UN3556, not the legacy generic UN3171; apply documentation/SOC details per the current EV shipping Fact Sheet and the carrier (see the EV ocean-compliance guide) EV shipping documents per the current Fact Sheet; cluster/telematics language.

# Request a Current Export Quotation

No static export price and no China MSRP were captured; none is invented. Enquire with Destination Country · Battery Build · Quantity · LHD/RHD · Destination Port · Incoterm.

# Frequently Asked Questions

What is the V6E's cargo capacity? A 6 m³ cargo bay (about 1.6 m long reference), with a load-oriented leaf-spring rear axle. How far does it go on a charge? About 260 km (working condition) with the 35.2 kWh base pack; a newer ~51 kWh build is quoted at 355–375 km — confirm the build. What is the top speed? 90 km/h, electronically limited — a city/last-mile characteristic. Has the V6E been exported? Yes — an EU-spec batch was recorded shipped to Türkiye in Feb 2024; that export build is not automatically identical to the China-spec van. Which motor does it use? A Wolong-ZF TZ185XS022 permanent-magnet motor, 60 kW/220 N·m.

# Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| V6E reveal (motor/0–50/speed limit) | Truck Home / Toutiao | CN | http://m.toutiao.com/group/7064810099012895240/ | 2026-09-03 | CROSS_CHECKED | 60 kW/220 N·m, 90 km/h limit, leaf-spring axle |
| V6E EU-spec export to Türkiye (6 m³) | China Transport News (industry news media) | CN media (export record: TR) | https://www.zgjtb.com/m/2024-02/01/content_398656.html | 2026-09-03 | CROSS_CHECKED | Feb-2024 EU-spec Türkiye batch, 6 m³ (news record, not OEM/government; not VERIFIED) |
| Light-commercial report (35.2 kWh/260 km/6 m³) | Sohu Auto | CN | https://m.sohu.com/a/931378694_121823499/ | 2026-09-03 | CROSS_CHECKED | Base battery/range, cargo volume |
| 2026 V6E (51 kWh/355 km) | Sohu Auto database | CN | http://db.auto.sohu.com/model_6721/a/990472906_120063592 | 2026-09-03 | TIME_SENSITIVE | Newer-build battery/range |

Confidence note: the Türkiye export record is CROSS_CHECKED industry-news reporting (news/industry media is not VERIFIED — only OEM/government/regulator/standards/formal technical documents reach VERIFIED); treated strictly as a delivery record, not a model-alias proof. Base motor/range are cross-checked; the newer 51 kWh build is time-sensitive. China GB/T and EU-spec builds are kept distinct per MODEL_ALIAS / SOURCE_SCOPE rules.

# Editorial Review

Author / reviewer: [AutoBridge Export Sourcing Team](/authors/) · method per our [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-03

Reference market: China, Xingxiang V6E (Chinese-market reference; EU-spec export build kept separate)

Verification method: Cross-check + official export record; battery-year and market scope isolated

Quality gates: EDITORIAL_QA_PASS=PASS · FACT_SOURCE_PASS=PASS · SEO_PASS=PASS · CODEX_REVIEW_PASS=PENDING (independent Codex review) · PUBLISH_APPROVED=FALSE until Codex sign-off