# Shipping an Electric Vehicle by Sea: UN3171, Class 9, UN38.3 and the Documents a Booking Depends On

# SEO Metadata

> ⛔ R4 HOLD — FACT_SOURCE_PASS=FAIL · CODEX_REVIEW_PENDING=FALSE · PUBLISH_APPROVED=FALSE. IMDG 42-24 / UN3556 corrected classification Fact Sheet still pending; current text reflects the superseded blanket-UN3171 basis and must be rewritten after Research update, incl. new URL /guides/ev-shipping-un3556-imdg-compliance/ (R4). Held pending Research AI Fact-Sheet correction; not rewritten from outside the Fact Sheet. Do not publish or send to Codex until re-verified.

SEO Title: EV Shipping Compliance: UN3171, IMDG Class 9, UN38.3 & MSDS Guide

Meta Description: How lithium-battery vehicles are classified for ocean freight (UN3171, IMDG Class 9), the SP390/P910 provisions, UN38.3 and GHS MSDS paperwork, China's 2025 lithium-shipping rule, and carrier-specific checks.

Suggested URL: /guides/ev-shipping-un3171-imdg-compliance/

H1: Shipping Electrified Vehicles by Sea: Dangerous-Goods Classification, Documents and Red Lines

Primary Keyword: ev shipping un3171 imdg dangerous goods requirements

Secondary Search Terms: UN3171 battery powered vehicle, IMDG class 9 vehicle, UN38.3 test report, MSDS GHS electric car shipping, SP390 P910, EV ocean freight SOC requirement

Internal Link Suggestions: /guides/china-vehicle-export-license-customs-documents/ ; /guides/container-vehicle-loading-lashing/ ; /vehicles/byd-dolphin/

Image Suggestions: UN number/class label diagram; document chain flow; Class 9 placard; booking-time decision tree

ALT Suggestions: "UN3171 IMDG class 9 electric vehicle"; "EV shipping document chain"

# Start Here: an EV Is Dangerous Goods, Even When It Looks Like a Car

A vehicle powered by a lithium battery is classified for sea transport under UN3171 "battery-powered vehicle/equipment," within IMDG Code Class 9 (miscellaneous dangerous goods). A hybrid carrying a traction battery is treated on the same logic. Treating an EV as ordinary RoRo/container cargo is the root cause of refused bookings and rollovers. This guide maps classification, provisions, documents and the items that change over time; the IMDG amendment in force and each carrier's extra rules must be re-confirmed at booking.

# Classification and Special Provisions

UN3171 / Class 9 is the entry for a battery-powered vehicle (cross-checked against Chinese maritime-safety material).

Special Provision 390 addresses vehicles transported with the battery installed (vehicle-and-battery together).

Packing Instruction P910 allows a whole vehicle to be carried without conventional dangerous-goods packaging (i.e., the vehicle itself is the "article"), subject to its conditions.

These provisions are why an intact, factory-built EV can ship as a vehicle rather than being crated as loose cells — but the relief is conditional, not blanket.

# The Document Set a Booking Rests On

UN38.3 test report — covering both the cells and the battery pack/module; a cell-only report is insufficient for the pack.

GHS-compliant MSDS/SDS — identifying UN3171, Class 9, Packing Group III and the EmS schedule.

Dangerous-goods declaration and carrier booking forms completed to the IMDG standard.

Loose/spare batteries are a different case — separate lithium-battery entries apply and a dangerous-goods packaging certificate (危包证) is required for loose batteries; do not assume the whole-vehicle relief covers spares packed alongside.

For damaged or defective batteries, separate entries such as UN3556 apply — such units need explicit carrier approval and often cannot move at all.

# Version and Timeliness (the moving parts)

UN38.3 revision: from 2026, Revision 7 and above is generally the accepted version (TIME_SENSITIVE — confirm the version your carrier and destination accept; older reports may be rejected).

China domestic rule: a technical requirement for ships carrying lithium batteries is reported to have taken effect 1 May 2025, refining declaration, inspection, packaging and stowage (single-source compliance analysis — confirm the current official text and how it applies to whole vehicles vs loose batteries).

IMDG amendment cycle: the amendment in force changes biennially with transition periods; state the applicable amendment on the booking.

# Carrier-Specific Layer (never assume the Code is the ceiling)

On top of IMDG, shipping lines impose their own conditions, commonly including:

A maximum state of charge (SOC) and proof of the 12 V/traction system state;

Whether the vehicle may ship by container or only RoRo, and terminal acceptance;

Firewatch/stowage and disconnect instructions; damaged/recalled-battery bans.

These vary by line and route and are TIME_SENSITIVE — obtain the carrier's current dangerous-goods acceptance in writing before promising a sailing.

# What This Guide Recommends You Prepare Before Booking

Cell + pack UN38.3 (current revision) and a GHS MSDS naming UN3171/Class 9/PG III/EmS.

A clear installed-battery vs loose-battery decision; arrange 危包证 for any spares.

The current IMDG amendment and China 2025 rule applicability confirmed in writing.

Carrier SOC/stowage/route conditions approved before container loading or RoRo delivery.

A damaged/defect screen (no warning lights, no impact/flood history) — feed this into the PSI (see pre-shipment inspection guide).

# Frequently Asked Questions

What UN number is an electric car shipped under? UN3171 (battery-powered vehicle), IMDG Class 9, when the battery is installed in the vehicle. Does the EV need dangerous-goods packaging? Under SP390/P910 a complete vehicle is not crated like loose cells, subject to conditions; loose batteries follow separate rules and need a packaging certificate. Is a cell-level UN38.3 report enough? No — both cells and the battery pack/module must be covered. Which UN38.3 version is accepted? Revision 7 and above is generally required from 2026; confirm with the carrier as this is time-sensitive. Can a damaged-EV battery ship normally? No — damaged/defective batteries fall under separate entries (e.g., UN3556) and need explicit approval or may be refused.

# Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| Lithium-battery transport (SP360/UN3171 explainer) | China MSA / Toutiao | CN / maritime | http://m.toutiao.com/group/6727329306692813316/ | 2026-09-03 | CROSS_CHECKED | UN3171, Class 9 classification context |
| UN3171 compliance (IMDG 40-20/SP390/P910) | Freight-forwarder material | Global | https://m.11467.com/product/d38134856.htm | 2026-09-03 | SINGLE_SOURCE | SP390, P910 provisions |
| Lithium-battery export documents (UN38.3/MSDS/包装证) | Dingzhou News | CN | https://www.dzxww.cn/article/293251787800392.shtml | 2026-09-03 | CROSS_CHECKED | UN38.3 cell+pack, GHS MSDS, loose-battery cert, UN3556, Rev7 |
| New-energy export customs risk (2025-05-01 rule) | Sohu compliance analysis | CN | https://m.sohu.com/a/1025837015_100159475/ | 2026-09-03 | SINGLE_SOURCE | China 2025 lithium-shipping requirement |

Scope note: classification/provisions are cross-checked; the China 2025 rule and SP/P-number detail are partly single-source, and the IMDG amendment, UN38.3 revision and carrier SOC rules are time-sensitive — confirm the official IMDG/UN text and the carrier's current terms at every booking.

# Editorial Review

Author / reviewer: [AutoBridge Export Sourcing Team](/authors/) · method per our [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-03

Reference market / scope: Global ocean dangerous-goods compliance (China rule flagged separately)

Verification method: Maritime-safety + forwarder cross-check; time-sensitive and single-source items isolated

Quality gates (R4 HOLD): EDITORIAL_QA_PASS=PASS · FACT_SOURCE_PASS=FAIL (IMDG 42-24 / UN3556 corrected Fact Sheet pending; uncertainty disclosed is not a substitute for verified core facts) · SEO_PASS=PASS · CODEX_REVIEW_PASS=FALSE (not eligible while FAIL) · PUBLISH_APPROVED=FALSE

Final Status = FACT_SOURCE_FAIL / AWAITING_RESEARCH — held for Research AI Fact-Sheet correction; not rewritten from outside the Fact Sheet, no reserve invoked.