# JMC Shunda (顺达) Light Truck: GVW, Payload, Engine and Cargo Body for Export Buyers

## SEO Metadata

SEO Title: JMC Shunda Light Truck Specs & Export Guide (GVW 4.27t, China-6)

Meta Description: Chinese-market JMC Shunda blue-plate light truck — JX493 diesel 90 kW, GVW 4.27 t, 3700 mm dropside bed, wheelbase and turning circle, plus the payload and engine-version checks exporters need.

Suggested URL: /vehicles/jmc-shunda/

H1: JMC Shunda: Chinese-Market Light-Truck Chassis, GVW and Cargo Specs for Export

Primary Keyword: jmc shunda light truck specs export

Secondary Search Terms: JMC Shunda JX493, 顺达小卡, Chinese light truck GVW, JMC dropside payload, China-6 diesel truck export

Internal Link Suggestions: /guides/vehicle-export-hs-code-classification/ ; /guides/emission-fuel-compatibility-china-vehicles/ ; /vehicles/dongfeng-tianjin-kr/

Image Suggestions: Shunda dropside side; cargo bed; JX493 engine; chassis frame; cab

ALT Suggestions: "JMC Shunda blue-plate light truck dropside"; "Shunda 3700mm cargo bed GVW 4.27t"

## Buyer Context: a Commercial Vehicle Page Is Built Around GVW and Payload

For a light truck the buyer is not comparing trim levels — they are matching GVW, rated payload, engine version and cargo body to an operating job. The Shunda (顺达) is JMC's China-6 blue-plate light truck line. The main order risk is engine-version mixing: the documented small-truck engine is the JX493ZLQ6D 2.499 L, while the wider Shunda range also uses a 2.8 L 493 with different outputs. This page documents the Chinese-market small-truck reference (REFERENCE MARKET: CHINA); no OEM primary sheet was captured, so figures are cross-checked trade-database values to confirm against the MIIT announcement and the vehicle certificate.

## Commercial Core: GVW, Payload and Cargo Body

| Item | Shunda small truck (China reference) |
| --- | --- |
| Engine | JX493ZLQ6D diesel, 2.499 L (do not merge with the 2.8 L 493) |
| Power | 90 kW (≈122 PS); a separate 116 PS version exists — match to trim |
| Dropside bed (L×W×H) | 3700 × 1850 × 380 mm |
| Gross vehicle weight (GVW) | 4.27 t |
| Curb weight | about 2.145 t (the 3.7 m box-van version is about 2.435 t) |
| Rated payload | about 1.93–2.0 t — final value per the MIIT announcement / certificate |
| Wheelbase | 2800 mm |
| Turning circle | 5.5 m |
| Emission | China-6 diesel |

Payload is the figure that most often causes compliance problems: a marketing "about two-tonne" line is not a legal rating. The authoritative rated payload is the one on the MIIT public announcement and the vehicle's certificate of conformity, and it changes with body type (dropside vs box van) and curb weight.

## Engine-Version Boundary

JX493ZLQ6D (this page): 2.499 L, 90 kW/122 PS.

A 2.8 L 493 and a 116 PS tune exist elsewhere in the Shunda line; quoting 116 PS for a 90 kW unit — or a 2.8 L displacement for a 2.499 L engine — is a configuration error.

Curb weight differs between dropside and box-van bodies, which feeds directly into payload; never reuse one body's payload for another.

## Chassis and Body-Builder Note

The 2800 mm wheelbase and 5.5 m turning circle suit urban distribution. Where a buyer adds a box, refrigerated unit or other superstructure, the final GVW/payload must be re-stated for the completed vehicle — the chassis figure alone is not the finished-truck rating.

## Overseas Buyer Verification Before Payment

For a light truck the 4×2 (or other) drive form is a certificate field that affects upfit and loading — confirm it per chassis announcement; the reference prose fixes GVW/payload but leaves drive form to the certificate.

Exact engine code/displacement/power against the VIN (2.499 L/90 kW vs 2.8 L/116 PS).

Rated payload and GVW from the MIIT announcement and certificate, not a brochure round-number.

Body type (dropside/box/special) and its curb weight; completed-vehicle rating for any upfit.

Diesel fuel sulphur and after-treatment compatibility per the emission/fuel guide; China-6 is not itself overseas approval.

HS classification (87.04 goods vehicle; special-purpose bodies may move headings) per the HS-code guide.

## Request a Current Export Quotation

No export price is published. Enquire with Destination Country · Quantity · Engine Version · Body Type · LHD/RHD · Destination Port · Incoterm.

## Frequently Asked Questions

What is the Shunda's GVW and payload? GVW 4.27 t with rated payload around 1.93–2.0 t; confirm the exact payload on the MIIT announcement/certificate for the specific body.

Which engine does this page cover? The JX493ZLQ6D 2.499 L diesel at 90 kW (122 PS); a 2.8 L/116 PS version is a different unit.

Why does curb weight matter? It sets usable payload and changes between dropside and box-van bodies — do not share one payload across bodies.

Is the China-6 truck approved for my market? Not automatically; destination emission and type approval are separate.

## Image Record

IMAGE_ASSET_PATH: none secured in repository

ORIGINAL_IMAGE_URL: not captured

SOURCE_PAGE: not captured

RIGHTS_HOLDER: unconfirmed

LICENSE: none secured — no third-party image may be published until rights are cleared

CHECKED_DATE: 2026-09-05

MODEL_TOPIC_MATCH: must match the exact model/version (or the guide topic) and reference market above

IMAGE_RIGHTS_STATUS: FAIL (no licensed asset captured; a placeholder or “retain old image” note is not accepted)

ALT by language:

EN: AutoBridge export-buyer reference — JMC Shunda, petrol/diesel Chinese-market vehicle / light truck

FR: Référence AutoBridge pour acheteurs export — JMC Shunda, véhicule thermique (marché chinois) / camion léger

DE: AutoBridge-Referenz für Exportkäufer — JMC Shunda, Verbrenner (chinesischer Markt) / Leicht-Lkw

ES: Referencia AutoBridge para compradores de exportación — JMC Shunda, vehículo de combustión (mercado chino) / camión ligero

PT: Referência AutoBridge para compradores de exportação — JMC Shunda, veículo a combustão (mercado chinês) / caminhão leve

JA: AutoBridge 輸出バイヤー向けリファレンス｜JMC Shunda, 中国市場仕様 内燃機関車 / 小型トラック

KO: AutoBridge 수출 바이어 참고 자료｜JMC Shunda, 중국 시장 내연기관 차량 / 경트럭

VI: Tài liệu tham khảo AutoBridge cho người mua xuất khẩu — JMC Shunda, xe động cơ đốt trong (thị trường Trung Quốc) / xe tải nhẹ

TH: เอกสารอ้างอิง AutoBridge สำหรับผู้ซื้อเพื่อการส่งออก — JMC Shunda, รถเครื่องยนต์สันดาป (ตลาดจีน) / รถบรรทุกขนาดเล็ก

ID: Referensi AutoBridge untuk pembeli ekspor — JMC Shunda, kendaraan mesin pembakaran (pasar Tiongkok) / truk ringan

AR: مرجع AutoBridge لمشتري التصدير — JMC Shunda, مركبة بمحرك احتراق (سوق الصين) / شاحنة خفيفة

ZH: AutoBridge 出口采购参考｜JMC Shunda, 中国市场燃油车 / 轻卡

## Sources & Verification

| Source title | Organization | Market | URL | Checked | Confidence | Supported facts |
| --- | --- | --- | --- | --- | --- | --- |
| Shunda small truck JX493ZLQ6D 122PS compliant 2t | 360che (Toutiao) | CN | http://m.toutiao.com/group/7296380434664292918/ | 2026-09-04 | CROSS_CHECKED | Engine, power, GVW, payload framing |
| Shunda small truck bed, GVW, curb weight | 360che (Toutiao) | CN | http://m.toutiao.com/group/6854066453151384075/ | 2026-09-04 | CROSS_CHECKED | 3700 bed, 4.27 t, curb weight |
| Shunda cargo 3.7m 116PS (in-sale) | 58.com trucks | CN | https://m.58.com/sh/huochec/63510160324146x.shtml | 2026-09-04 | SINGLE_SOURCE | Alternate 116 PS version (excluded) |
| Shunda wheelbase / turning circle | 360che (Toutiao) | CN | http://m.toutiao.com/group/6815508461468516878/ | 2026-09-04 | CROSS_CHECKED | 2800 mm WB, 5.5 m turning |
| Road motor-vehicle manufacturers & products catalog — public query system | MIIT, China Ministry of Industry and Information Technology | CN | https://service.miit-eidc.org.cn/miitxxgk/gonggao_xxgk/index_ggcp.html | 2026-09-05 | VERIFIED | Verify the manufacturer, approved model and homologation (公告) catalog entry before ordering or export |
| GB 16735-2019 Road vehicles — Vehicle identification number (VIN) requirements | SAMR national standards full-text platform | CN | https://openstd.samr.gov.cn/bzgk/std/newGbInfo?hcno=E2EBF667F8C032B1EDFD6DF9C1114E02 | 2026-09-05 | VERIFIED | VIN structure and nameplate data fields used in the buyer's VIN/nameplate verification |
| GB 7258 motor-vehicle safety technical conditions (GB 7258-2017 current; GB 7258-2026 applies from 2027-07-01) | SAMR, State Administration for Market Regulation | CN | https://www.samr.gov.cn/xw/sj/art/2026/art_9b2b3fa425e3461e8dc3bd270b410ceb.html | 2026-09-05 | VERIFIED | Baseline in-China vehicle safety technical conditions and the current/revision timeline |
| GB 18352.6-2016 with amendment XG1-2026 (China-6 light-vehicle pollutant emissions) | Ministry of Ecology and Environment (MEE) | CN | https://www.mee.gov.cn/xxgk2018/xxgk/xxgk01/202604/t20260430_1150676.html | 2026-09-05 | VERIFIED | China-6 light-vehicle emission stage and the 2026 amendment implementation dates |
| GB 19147 automobile diesel fuel national standard (low-sulphur basis) | SAMR national standards full-text platform | CN | https://openstd.samr.gov.cn/bzgk/std/newGbInfo?hcno=88F31AEECC7F7AE17C5A99496E532D2A | 2026-09-05 | VERIFIED | Diesel fuel grade and sulphur basis to match engine/emission design overseas |

*Confidence note: no OEM primary sheet; trade-database cross-checks. Rated payload explicitly deferred to MIIT announcement/certificate; 2.499 L and 2.8 L engines kept separate. Chinese-market reference only.*

## Editorial Review

Author: AutoBridge Export Editorial Team · [Sourcing Team](/authors/) · method per our [Editorial Policy](/editorial-policy/)

Last reviewed: 2026-09-04 · Fact sheet version: v1.1-2026-09-04

Reference market: China, Shunda JX493ZLQ6D small truck (Chinese-market reference; payload per certificate; not a global spec)

Verification method: Trade-database cross-check; engine versions separated; legal rating deferred to official announcement

Transparency: AI_ASSISTED=true · HUMAN_REVIEWED=false · evidence.first_hand=false · PUBLISH_APPROVED=false

Quality gates: EDITORIAL_QA_PASS=PASS · FACT_SOURCE_PASS=PASS · SEO_PASS=PASS · CODEX_REVIEW_PASS=PENDING · PUBLISH_APPROVED=FALSE until independent sign-off

Tags: #JMCShunda #LightTruck #GVWPayload #China6Diesel #CommercialVehicleExport