# Forthing Lingzhi M5 2024 — Work Van / Commercial MPV: Seating Layouts, Powertrains and Export Checks

# SEO Metadata

SEO Title: Forthing Lingzhi M5 2024 Van Specs, Seating & CNG/Petrol Export Guide

Meta Description: 2024 Dongfeng Forthing Lingzhi M5 (M5L) China specs: 5/7/9-seat layouts, two wheelbases, 1.5T petrol 6MT and 1.6L CNG 5MT, dimensions, and commercial-buyer verification points.

Suggested URL: /vehicles/forthing-lingzhi-m5/

H1: Forthing Lingzhi M5 / M5L 2024 (China): A Utilitarian Van Spec Map for Commercial Buyers

Primary Keyword: Forthing Lingzhi M5 van specifications export

Secondary Search Terms: Dongfeng Lingzhi M5L 9-seat, Forthing M5 CNG 1.6, Chinese commercial MPV van export, Lingzhi M5 wheelbase, Dongfeng passenger van

Internal Link Suggestions: /vehicles/gac-trumpchi-m8-2024/ ; /guides/commercial-truck-fleet-procurement-china/ ; /guides/right-hand-drive-chinese-cars/

Image Suggestions: side profile M5L long body; 9-seat interior layout; 1.5T vs 1.6 CNG card; cargo/seat folding; sliding side door

ALT Suggestions: "2024 Forthing Lingzhi M5L nine-seat commercial van, China spec"; "Lingzhi M5 5/7/9 seat layout options"; "Lingzhi M5 1.5T petrol versus 1.6 CNG"

# Vehicle Role

The Lingzhi M5 is a utilitarian front-wheel-drive van / commercial MPV built for people-moving and light multi-use duty rather than premium comfort. It matters to importers, shuttle operators and small-fleet owners who need many seats and a large box at a low acquisition cost. The reference line is the 2024 M5L 1.5T 舒适型 9-seater (REFERENCE MARKET: CHINA); a shorter M5 and a CNG (compressed natural gas) version also exist. Forthing (Dongfeng Liuzhou) already sells in some RHD markets such as Thailand under other nameplates (e.g., Friday/V9), but those are different products — Lingzhi M5 export specs must be confirmed per destination and not inferred from the Chinese line or from Forthing's other overseas vans.

# Body Sizes and Wheelbases

| Item | M5L (long) | M5 (short) |
| --- | --- | --- |
| Length × Width × Height | 5135 × 1720 × 1970 mm | Same width/height, shorter body |
| Wheelbase | 3000 mm | 2800 mm |
| Seat layouts offered | 7 / 9 seats | 5 / 7 / 8 seats |

Dimensions, wheelbases and seat layouts are VERIFIED against the Forthing official parameter page and matched by the databases. The tall, near-vertical body (1970 mm height) is a deliberate cargo/people box — useful interior volume is the design goal. Confirm the exact seat count, because the same "M5L" label covers both 7- and 9-seat builds.

# Powertrains — Petrol Turbo vs Naturally Aspirated CNG

| Powertrain | Engine | Power / torque | Transmission |
| --- | --- | --- | --- |
| 1.5T petrol (M5L) | Turbo L4 | 140 kW (190 PS) / 300 N·m | 6-speed manual |
| 1.6L CNG | Naturally aspirated L4 (natural gas) | 75 kW (102 PS) / 140 N·m | 5-speed manual |

Two procurement points stand out. First, both lines are manual — there is no automatic in the captured 2024 range, so buyers needing auto/AMT must look elsewhere or confirm a specific export build. Second, the CNG version is substantially lower-powered (75 kW) and is relevant only where natural-gas refuelling and CNG registration are supported; for gas-infrastructure markets it can cut running costs, but it is not a like-for-like substitute for the 1.5T. The 1.5T records a WLTC combined figure around 8.4 L/100 km and a 160 km/h top speed. Curb weight for the M5L 1.5T 9-seater is ~1700 kg (single source, Dongchedi — confirm on the compliance plate).

# Seating and Use Configuration

Because this is a commercial people/cargo platform, specify the interior as carefully as the engine:

Passenger shuttle: 9-seat M5L maximises fare-paying capacity — verify local rules on which vehicle categories may carry 9 occupants and the licence class required.

Crew + cargo / convertible use: confirm whether seats are removable/folding and whether the destination registers the unit as passenger or commercial (this affects taxation and inspection).

Access: confirm sliding side door count and tailgate type for your operation.

Air-conditioning: rear-cabin AC for 9-seat duty should be confirmed by grade.

Payload and cargo-volume ratings were not captured in Stage 1 and are not estimated here; request them from the official commercial specification for the exact wheelbase/seat build.

# China Reference vs Export Reality

Chinese units are LHD, manual, China-certified. Confirm whether a factory RHD or an export-specific Lingzhi M5 exists for the destination rather than assuming conversion.

Verify the destination's emission and fuel/CNG rules: a CNG car needs recognised gas-tank certification and refuelling standards, and a China petrol calibration must meet local emission acceptance.

Confirm spare-parts and clutch/gearbox support for a manual commercial van, plus warranty (China reference: five years / 100,000 km — domestic coverage, distributor-dependent overseas).

Check whether the destination treats a 9-seater as a minibus requiring additional driver/operator licensing.

# Pre-Payment Verification List

Pin down M5 vs M5L (2800 vs 3000 mm wheelbase) and exact seat count (5/7/8/9).

Pin down powertrain (1.5T petrol 6MT vs 1.6L CNG 5MT); confirm manual is acceptable for the end customer.

For CNG, verify gas-cylinder certification, refuelling standard and registration class at destination.

Confirm LHD/RHD, emission acceptance, rear AC and door configuration by grade.

Obtain official payload/cargo figures and the VIN plate curb weight before registration.

# Request a Current Export Quotation

AutoBridge quotes on request rather than publishing fixed export prices. Chinese domestic references — M5L 1.5T around RMB 74,900–96,800 and CNG from about RMB 67,900 — are domestic, time-sensitive references, not export prices. Enquire with Destination Country · Quantity · M5/M5L & Seats · Petrol/CNG · LHD/RHD · Destination Port · Incoterm.

# Buyer FAQ

Does the Lingzhi M5 come with an automatic gearbox? In the captured 2024 China range, no — the 1.5T is 6-speed manual and the 1.6 CNG is 5-speed manual. How many seats can it have? The long M5L offers 7 or 9 seats; the short M5 offers 5/7/8 — confirm the exact layout. What is the CNG version for? It runs on compressed natural gas to lower fuel cost where CNG infrastructure and certification exist, but at 75 kW it is much less powerful than the 140 kW 1.5T. What is the wheelbase difference? 3000 mm (M5L long) versus 2800 mm (M5 short). Is it sold in right-hand drive? Forthing sells other products in some RHD markets, but an RHD Lingzhi M5 must be confirmed for the specific destination rather than assumed.

# Sources & Verification

| Source title | Organization | Market | URL | Checked | Supported facts |
| --- | --- | --- | --- | --- | --- |
| Lingzhi official parameters | Dongfeng Forthing (official) | CN | https://www.fxauto.com.cn/index.php/car/parameter?cid=37 | 2026-09-02 | Wheelbase 2800/3000, 5/7/9 seats, chassis framework |
| M5L 1.5T 9-seat model | Dongchedi (懂车帝) | CN | https://www-hl.dongchedi.com/auto/series/445/model-92416 | 2026-09-02 | 1.5T 140 kW/300 N·m, 3000 mm WB, curb 1700 kg |
| Lingzhi parameter table | PCauto (太平洋汽车) | CN | https://car.pcauto.com.cn/360/m124835/config.html | 2026-09-02 | Dimensions, CNG powertrain, WLTC 8.4, warranty, price |
| Lingzhi M5 article | Yiche (易车) | CN | https://m.yiche.com/zhai/261903/ | 2026-09-02 | Dimensions, wheelbase, 7/9-seat layouts |

Verification note: dimensions/wheelbase/seats VERIFIED (official + databases); 1.5T/CNG powertrains CROSS_CHECKED; curb weight SINGLE_SOURCE. Payload/cargo volume not in fact base — to be confirmed officially. Export/RHD specs not asserted.

# Editorial Review

Reviewed by: AutoBridge Export Sourcing Team

Last reviewed: 2026-09-02

Reference market: China, 2024 M5/M5L

Verification method: Brand-official parameter page plus database cross-check; manual/CNG and single-source items labelled

Content status: QA_PASS (Master/EN)